To: INCITS Technical Committee T10

From: Kevin Butt and Sivan Tal, IBM Corporation

Date: July 30, 2026

Document: T10/08-101r0 :  SPC-4 – CbCS field byte alignment changes

Introduction

Several fields in CbCS structures are not aligned for efficient use. In general, aligning fields of size X to X byte boundary makes implementations more efficient.

Key:

Added or changed text

Deleted text

{{Proposer’s note}}

Revision History

r0	Initial revision

Proposed changes in SPC-4

7.6.x	Capabilities CbCS page

Table x1 the format of the Capabilities CbCS page.

{{Added some reserved field padding to make all the 4 byte fields 4 byte aligned.

The total addition is 4 bytes plus 2 bytes per supported CbCS method.}}

Table x1 – Capabilities CbCS page format

| Bit Byte | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | (MSB) | PAGE CODE (0010h) |  |  |  |  |  |  |
| 1 |  |  |  |  |  |  |  | (LSB) |
| 2 | (MSB) | PAGE LENGTH (i*4+j*4+k*4+12) |  |  |  |  |  |  |
| 3 |  |  |  |  |  |  |  | (LSB) |
| 4 | GKS | LUKS | GCMS | LUCMS | Reserved |  |  |  |
| 5 | Reserved |  |  |  |  |  |  |  |
| 6 | (MSB) | Number of supported CbCS methods (i) |  |  |  |  |  |  |
| 7 |  |  |  |  |  |  |  | (LSB) |
| 8 |  | Reserved (first) |  |  |  |  |  |  |
| 9 |  |  |  |  |  |  |  |  |
| 10 | (MSB) | SUPPORTED CBCS METHOD (first) |  |  |  |  |  |  |
| 11 |  |  |  |  |  |  |  | (LSB) |
|  | . . . |  |  |  |  |  |  |  |
| i*4+4 | (MSB) | Reserved (last) |  |  |  |  |  |  |
| i*4+5 |  |  |  |  |  |  |  | (LSB) |
| i*4+6 |  | SUPPORTED CBCS METHOD (last) |  |  |  |  |  |  |
| i*4+7 |  |  |  |  |  |  |  |  |
| i*4+8 |  | Reserved |  |  |  |  |  |  |
| i*4+9 |  |  |  |  |  |  |  |  |
| i*4+10 | (MSB) | Number of supported integrity check value algorithms (j) |  |  |  |  |  |  |
| i*4+11 |  |  |  |  |  |  |  | (LSB) |
| i*4+12 | (MSB) | SUPPORTED INTEGRITY CHECK VALUE ALGORITHM (first) |  |  |  |  |  |  |
| i*4+15 |  |  |  |  |  |  |  | (LSB) |
|  | . . . |  |  |  |  |  |  |  |
| i*4+j*4+8 | (MSB) | SUPPORTED INTEGRITY CHECK VALUE ALGORITHM (last) |  |  |  |  |  |  |
| i*4+j*4+11 |  |  |  |  |  |  |  | (LSB) |
| i*4+j*4+12 |  | Reserved |  |  |  |  |  |  |
| i*4+j*4+13 |  |  |  |  |  |  |  |  |
| i*4+j*4+14 | (MSB) | Number of supported D-H groups (k) |  |  |  |  |  |  |
| i*4+j*4+15 |  |  |  |  |  |  |  | (LSB) |
| i*4+j*4+16 |  | SUPPORTED D-H GROUP (first) |  |  |  |  |  |  |
| i*4+j*4+19 |  |  |  |  |  |  |  | (LSB) |
|  | . . . |  |  |  |  |  |  |  |
| i*4+j*4+k*4+12 |  | SUPPORTED D-H GROUP (last) |  |  |  |  |  |  |
| i*4+j*4+k*4+15 |  |  |  |  |  |  |  | (LSB) |

7.6.y	Attributes CbCS page

Table x2 specifies the format of the Attributes CbCS page.

{{Added ‘reserved’ fields to make fields byte aligned for their size.}}

{{Do we want to make the key identifier 8 byte aligned? Or can we assume they’re strings?}}

Table x2 – Attributes CbCS page format

| Bit Byte | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | (MSB) | PAGE CODE (0010h) |  |  |  |  |  |  |
| 1 |  |  |  |  |  |  |  | (LSB) |
| 2 | (MSB) | PAGE LENGTH (n-3) |  |  |  |  |  |  |
| 3 |  |  |  |  |  |  |  | (LSB) |
| 4 | (MSB) | CBCS METHOD |  |  |  |  |  |  |
| 5 |  |  |  |  |  |  |  | (LSB) |
| 6 |  | Reserved |  |  |  |  |  |  |
| 7 |  |  |  |  |  |  |  |  |
| 8 | (MSB) | POLICY ACCESS TAG |  |  |  |  |  |  |
| 11 |  |  |  |  |  |  |  | (LSB) |
| 12 |  | Reserved |  |  |  |  |  |  |
| 15 |  |  |  |  |  |  |  |  |
| 16 | (MSB) | MASTER KEY IDENTIFIER |  |  |  |  |  |  |
| 23 |  |  |  |  |  |  |  | (LSB) |
| 24 | (MSB) | WORKING KEY IDENTIFIER 0 |  |  |  |  |  |  |
| 31 |  |  |  |  |  |  |  | (LSB) |
|  | . . . |  |  |  |  |  |  |  |
| 144 | (MSB) | WORKING KEY IDENTIFIER 15 |  |  |  |  |  |  |
| 151 |  |  |  |  |  |  |  | (LSB) |
| 152 | (MSB) | CLOCK |  |  |  |  |  |  |
| 157 |  |  |  |  |  |  |  | (LSB) |
| 158 | Reserved |  |  |  |  |  |  |  |
| 159 | SECURITY TOKEN LENGTH |  |  |  |  |  |  |  |
| 160 | (MSB) | SECURITY TOKEN |  |  |  |  |  |  |
| n |  |  |  |  |  |  |  | (LSB) |

7.6.z	Set attributes CbCS page

Table x3 specifies the format of the Set Attributes CbCS page.

{{Added 2 reserved bytes to make POLICY ACCESS TAG 4 byte aligned.}}

Table x3 – Set attributes CbCS page format

| Bit Byte | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | (MSB) | PAGE CODE (0011h) |  |  |  |  |  |  |
| 1 |  |  |  |  |  |  |  | (LSB) |
| 2 | (MSB) | PAGE LENGTH (8) |  |  |  |  |  |  |
| 3 |  |  |  |  |  |  |  | (LSB) |
| 4 | (MSB) | CBCS METHOD |  |  |  |  |  |  |
| 5 |  |  |  |  |  |  |  | (LSB) |
| 6 |  | Reserved |  |  |  |  |  |  |
| 7 |  |  |  |  |  |  |  |  |
| 8 | (MSB) | POLICY ACCESS TAG |  |  |  |  |  |  |
| 11 |  |  |  |  |  |  |  | (LSB) |

7.6.w	Set Key CbCS page

Table x4 specifies the Set Key CbCS page format.

{{Added 2 reserved bytes to make KEY IDENTIFIER 8 byte aligned.}}

Table x4 - Key CbCS page format

| Bit Byte | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | (MSB) | PAGE CODE (0012h) |  |  |  |  |  |  |
| 1 |  |  |  |  |  |  |  | (LSB) |
| 2 | (MSB) | PAGE LENGTH (32) |  |  |  |  |  |  |
| 3 |  |  |  |  |  |  |  | (LSB) |
| 4 |  | Reserved |  |  |  |  |  |  |
| 6 |  |  |  |  |  |  |  |  |
| 7 | Reserved |  |  |  | KEY VERSION |  |  |  |
| 8 |  | KEY IDENTIFIER |  |  |  |  |  |  |
| 15 |  |  |  |  |  |  |  |  |
| 16 |  | SEED |  |  |  |  |  |  |
| 35 |  |  |  |  |  |  |  |  |

7.6.v	Set Master Key, Seed Exchange CbCS page

Table x5 specifies the Set Master Key, Seed Exchange CbCS page format.

{{Added 2 reserved bytes to make DH DATA LENGTH 4 byte aligned.}}

Table x5 - Set Master Key, Seed Exchange CbCS page format

| Bit Byte | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | (MSB) | PAGE CODE (0013h) |  |  |  |  |  |  |
| 1 |  |  |  |  |  |  |  | (LSB) |
| 2 | (MSB) | PAGE LENGTH (n-3) |  |  |  |  |  |  |
| 3 |  |  |  |  |  |  |  | (LSB) |
| 4 | (MSB) | DH GROUP |  |  |  |  |  |  |
| 5 |  |  |  |  |  |  |  | (LSB) |
| 6 |  | Reserved |  |  |  |  |  |  |
| 7 |  |  |  |  |  |  |  |  |
| 8 | (MSB) | DH DATA LENGTH |  |  |  |  |  |  |
| 11 |  |  |  |  |  |  |  | (LSB) |
| 12 |  | DH DATA |  |  |  |  |  |  |
| n |  |  |  |  |  |  |  |  |

7.6.u	Set Master Key, Change Master Key CbCS page

Table x6 specifies the format of the Set Master Key, Change Master Key CbCS page.

{{Added 4 reserved bytes to make KEY IDENTIFIER 8 byte aligned.}}

Table x6 – Set Master Key, Change Master Key CbCS page format

| Bit Byte | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | (MSB) | PAGE CODE (0014h) |  |  |  |  |  |  |
| 1 |  |  |  |  |  |  |  | (LSB) |
| 2 | (MSB) | PAGE LENGTH (n-3) |  |  |  |  |  |  |
| 3 |  |  |  |  |  |  |  | (LSB) |
| 4 |  | Reserved |  |  |  |  |  |  |
| 7 |  |  |  |  |  |  |  |  |
| 8 | (MSB) | KEY IDENTIFIER |  |  |  |  |  |  |
| 15 |  |  |  |  |  |  |  | (LSB) |
| 16 | (MSB) | APPLICATION CLIENT DATA LENGTH (k-19) |  |  |  |  |  |  |
| 19 |  |  |  |  |  |  |  | (LSB) |
| 20 |  | APPLICATION CLIENT DH DATA |  |  |  |  |  |  |
| k |  |  |  |  |  |  |  |  |
| k+1 | (MSB) | DEVICE SERVER DATA LENGTH (n-(k+4)) |  |  |  |  |  |  |
| k+4 |  |  |  |  |  |  |  | (LSB) |
| k+5 |  | DEVICE SERVER DH DATA |  |  |  |  |  |  |
| n |  |  |  |  |  |  |  |  |

6.x.2	CbCS capability format

The format of the CbCS capability descriptor is defined in Table x7.

{{Reordered fields to improve alignment.}}

Table 20 - Capability descriptor format

| Bit Byte | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | DESIGNATION TYPE |  |  |  | KEY VERSION |  |  |  |
| 1 | CBCS METHOD |  |  |  |  |  |  |  |
| 2 | (MSB) | INTEGRITY CHECK VALUE ALGORITHM CAPABILITY EXPIRATION TIME |  |  |  |  |  |  |
| 7 |  |  |  |  |  |  |  | (LSB) |
| 8 | (MSB) | CAPABILITY EXPIRATION TIME INTEGRITY CHECK VALUE ALGORITHM |  |  |  |  |  |  |
| 11 |  |  |  |  |  |  |  | (LSB) |
| 12 |  | PERMISSIONS BIT MASK descriptor |  |  |  |  |  |  |
| 15 |  |  |  |  |  |  |  |  |
| 16 | (MSB) | POLICY ACCESS TAG |  |  |  |  |  |  |
| 19 |  |  |  |  |  |  |  | (LSB) |
| 20 |  | Designation descriptor |  |  |  |  |  |  |
| 55 |  |  |  |  |  |  |  |  |
| 56 |  | DISCRIMINATOR |  |  |  |  |  |  |
| 71 |  |  |  |  |  |  |  |  |

END OF DOCUMENT