TO: 		ATA Local Bus Working Group			X3T10/94-027r2

FROM:	R. Kalish 	408-957-7169

A. Pham	408-957-4804

DATE:		January 20, 1994

This information was presented at the December ATA Local Busworkinggroup meeting.  Although there was only a very limited time spentdiscussingit, the main objection voiced was that this was un-necessary andcould beachieved currently using a read-long command.  Hopefully moretime can bespent during the next meeting, and we can discuss why this cannot be donewith the read-long command and why this scheme will increase data transfer integrity.  We do not believe that the existing ATA protocol will be violated or required to change because even though the status is read beforethe data is transferred on a host read, when the host determines that the data it receivedis corrupted, the host should just execute another read command.  The drive may very well have sent good data, but the data was corrupted during thetransfer.

============================================================

Proposal for Data CRC across the Cable

ATA PIO Mode 4 and Multi-word DMA Mode 2 need a reliable method to verify the data transferred across the cable was received properly and was not corrupted.  This feature has long been a part of the SCSI interface with the parity bit enabling the verification of each byte of data.  Since there is no room on the cable for parity on each data transfer, Al and I would like to propose a method for data integrity verification across the cable.

This would mean adding 4 new commands for Read/Write-Multiple W/CRC and Read/WriteDMA W/CRC and only be for the high speed modes (20 Mbytes/sectransferrates - or whatever speed PIO Mode 4 becomes). Since the current Read/Write-Multiple and DMA commands are optional, these new commands would also be optional.  We are proposing the following:

1.   READ Multiple W/CRC - Command Code CCh

The new command for Read Multiple W/CRC will transfer 516 bytesof data per sector from the drive to the host.

2.   WRITE Multiple W/CRC - Command Code CDh

The new command for Write Multiple W/CRC will transfer 516 bytes of data per sector from the host to the drive.  A bad transfer will result in an error flag in both status registers for the status of the operation.

3.   READ Multiple DMA W/CRC - Command Code CEh

The new command for Read Multiple DMA W/CRC will transfer 516bytes ofdata per sector from the drive to the host.  This command will abort if the drive is not set up for Multiple Mode DMA.

4.   WRITE Multiple DMA W/CRC - Command Code CFh

The new command for Write Multiple DMA W/CRC will transfer 516bytes of data per sector from the host to the drive.  A bad transfer will result in anerror flag in both status registers for the status of the write operation. This command will abort if the drive is not set up for Multiple Mode DMA.

5.   CRC Polynomial

The 4 byte CRC polynomial is the same as used for P1394 (and manyotherspecifications) which should be universally acceptable (We willnot discussthe merits of this number since there are only a very few peoplewho fullyunderstand the pros and cons of this polynomial versus another,and we arenot in that group).

The CRC has the following properties:

Generator Polynomial:

G(x) =  x^32 + x^26 + x^23 + x^22 + x^16 + x^12 + x^11 +x^10 + x^8 + x^7 + x^5 + x^4 + x^2 + x + 1

CRC Register is preset to all 1's before generation and checking.

Generated CRC Redundancy Polynomial:

R$(x) = one's complement of the remainder  polynomial of degree less than 32,  i.e. the remainder is inverted before beingshifted out.

Check Polynomial produced by the receiver upon receiving anerror-free     sequence ( data and CRC redundancy ):

C(x)=x^31+x^30+x^26+x^25+x^24+x^18+x^15+x^14+x^12+x^11+x^10+x^8+x^6+x^5+x^4+x^3+x+16.

General Requirements

The drive must be previously set in the multiple mode or all ofthe above commands will abort.  These commands will be used on a sector by sector block of data. The process of generating and checking the CRCwould most likely be implemented with logic in a bridge adapter appending the CRC on a disk write and check the CRC on a disk read.  If there is a CRCerror when reading data from the disk, the host bridge logic will be required to notify the host of the error.  We can propose the exact method for this, or leave the implementation up to the bridge designer since this will require BIOS modification or Driver support anyway.  Our earlier proposal of letting the host microprocessor generate and check the CRC would probablycause a major time and resource burden to the system.

7.   Drive Requirements

The drive will need two bits in the Identify Drive Parameter Information to specify that the drive can support the Read/Write Multiple W/CRC and the Read/Write Multiple DMA W/CRC commands.  A likely place is in Word 49  bits 13 and 14 since this word is specifying drive capabilities,but there is no hard reason for not putting this information elsewhere.  We are assuming that if a drive will support a Read W/CRC that it must support the WriteW/CRC also, but the drive does not have to support both the PIOand DMA modes if it supports one.