NOTE: Copies of this document may be purchased from:				X3T10/94-168r0

Global Engineering Documents, 15 Inverness Way East,

Englewood, CO 80112-5704, Phone: (800) 854-7179,

Outside USA and Canada: (303) 792-2181, FAX: (303) 792-2192

SCSI-2 Technical Information Bulletin #1

Procedures for Logging Operations in SCSI-2

August 1, 1994

Abstract:  This document provides guidance in the use of the LOG SELECT and

LOG SENSE commands contained in American National Standard X3.131-1994, Small

Computer System Interface-2 (SCSI-2).  This document does not replace the

descriptions in SCSI-2 and is not intended to conflict with SCSI-2.  The

purpose of this document is to provide more information to gain a more

uniform implementation of the SCSI-2 logging functions.

NOTICE

Recognizing the need for a uniform approach to the responsibility for

disseminating interpretations to approved American National Standards, the

Accredited National Standards Committee X3 - Information Processing Systems

has authorized the publication of Technical Information Bulletins.

This Technical Information Bulletin is issued in response to questions which

have been raised regarding certain specifications in the content of:

X3.131-1994, American National Standard Small Computer System Interface-2

(SCSI-2).

This Bulletin was prepared by Technical Committee X3T10, which developed that

standard and was authorized for release by Committee X3 in order to provide

interpretations as quickly as possible in response to questions raised.

This Bulletin, while reflecting the technical opinion of the Technical

Committee which developed the standard, is intended solely as supplementary

information to other users of the standard.  That standard, ANS X3.131-1994,

as approved through the publication and voting procedures of the American

National Standards Institute, is not altered by this Bulletin.  Any

subsequent revision to the standard, ANS X3.131-1994, may or may not reflect the

contents of this Technical Information Bulletin.

Glossary

List Parameter -          A Parameter Value which consists of a string

of ASCII graphic codes

Log Page -                A page made up of one or more Log Parameters

Log Parameter -           Log information which is made up of a

Parameter Code, a Parameter Control Byte, and

a Parameter Value.

Parameter Code -          A unique identifier which is used to

distinguish between the different Log

Parameters within a single Log Page.

Parameter Control Byte -  Used to tell the target how to update, save,

use thresholds, determine format, etc. of the

Parameter Value.

Parameter Pointer Field - Contains a Parameter Code

Parameter Value -         A counter, cumulative, threshold, or ASCII

value

NV -                      Not Valid

x -                       The value of the bit or field is not relevant.

LOG SENSE Command

The LOG SENSE command may be used to do two functions:  One is to allow the

target to save the Log Parameters in a Log Page to non-volatile storage.  The

other is to allow the initiator to receive the value of the current Log

Parameters for a given Log Page.

The following lists the definitions of the LOG SENSE Command Descriptor Block

(CDB) fields:

LOG SENSE CDB Values

PPC  SP   PC

Bit  Bit  Field        Description

---  ---  -----        -------------------------------------------------------

0    -    --           Indicates that the log parameter requested from the

target shall begin with the parameter code specified by

the parameter pointer field in ascending order of

parameter codes from the specified log page.

1    -    --           Indicates that the target shall return a log page

consisting only of the log parameters in which a log

parameter value has changed since the last LOG SELECT

or LOG SENSE command.  The target shall return only

those log parameters following the parameter pointer

field.

-    0    --           Indicates the target shall perform the specified LOG

SENSE command and shall not save any log parameters.

-    1    --           Indicates that the target shall perform the specified

LOG SENSE command and shall save all log parameters

identified as savable by the DS bit to a non-volatile

vendor-specific location if allowed. (See the list

below to determine the interaction between the SP and

DS bits to see what 'allowed' means.)

-    -    00           Indicates the target shall return the Current Threshold

Values.

-    -    01           Indicates the target shall return the Current

Cumulative Values.

-    -    10           Indicates the target shall return the Default Threshold

Values.

-    -    11           Indicates the target shall return the Default

Cumulative Values.

The following lists the possible Parameter Values which can be returned:

Log Page

Parameter

Log Sense CDB   Control

Values          Byte Value   Target Action

-------------   ----------   -------------------------------------------------

PPC   PC        LP

Bit   Field     Bit          Parameter Values returned to the Initiator

---   -----     ---          -------------------------------------------------

0    00        0            Returns all Current Threshold Values starting

with the specified Parameter Pointer.

0    01        0            Returns all Current Cumulative Values starting

with the specified Parameter Pointer.

0    10        0            Returns all Default Threshold Values starting

with the specified Parameter Pointer.

0    11        0            Returns all Default Cumulative Values starting

with the specified Parameter Pointer.

1    00        0            Returns only the Current Threshold Values which

have changed starting with the specified

Parameter Pointer.

1    01        0            Returns only the Current Cumulative Values which

have changed starting with the specified

Parameter Pointer.

1    10        0            Returns only the Default Threshold Values which

have changed starting with the specified

Parameter Pointer.

1    11        0            Returns only the Default Cumulative Values which

have changed starting with the specified

Parameter Pointer.

0    xx        1            Returns all the List Parameters starting with the

specified Parameter Pointer.

1    xx        1            Returns only the List Parameters which have

changed starting with the specified Parameter

Pointer.

The following lists the possible save options for the LOG SENSE command.

The listed options define the save operations which occur as a direct result

of the LOG SENSE command.  Further save operations are a function of the TSD

bit in the Log Parameter Control Byte.

Log Page

Parameter

CDB         Control

Values      Byte Value   Target Action

----------  ----------   -----------------------------------------------------

SP   PC     DS   LP

Bit  Field  Bit  Bit

---  -----  ---  ---

0    xx     x    x       Do not save any of the Log Parameters into non-

volatile storage.

1    00     0    0       Save all the Current Threshold Values of the selected

Log Page into non-volatile storage.

1    01     0    0       Save all the Current Cumulative Values of the

selected Log Page into non-volatile storage.

1    10     0    0       Save all the Default Threshold Values of the selected

Log Page into non-volatile storage.

1    11     0    0       Save all the Default Cumulative Values of the

selected Log Page into non-volatile storage.

1    xx     0    1       Save all the current List Parameter Values of the

selected Log Page into non-volatile storage.

1    xx     1    x       Do not save any of the Log Parameters into non-

volatile storage.

Log Select Command

One function of the LOG SELECT command is to allow the initiator a method of

sending parameter values to the target.

The following lists the definitions of the LOG SELECT Command Descriptor Block

(CDB) fields:

LOG SELECT CDB Values

--------------------------

Parameter

PCR  SP   PC     List

Bit  Bit  Field  Length     Description

---  ---  -----  ---------  --------------------------------------------------

0    -    --       -        Indicates that the log parameters shall not be

reset.

1    x    xx     0000h      Indicates the target shall set all implemented

parameter values to the target-defined default

values.

1    x    xx     GT 0       An illegal condition.

-    0    --      -         Indicates the target shall not save any of of the

log parameters.

-    1    --      -         Indicates that after performing the specified LOG

SELECT operation the target shall save to non-

volatile memory all savable log parameters.  (See

the list above to determine the interaction

between the SP and DS bits to see what 'savable'

means.)

-    -    00      -         Indicates the initiator shall send Threshold

Values.

-    -    01      -         Indicates the initiator shall send Cumulative

Values.

-    -    10      -         Indicates the initiator shall send Default

Threshold Values.

-    -    11      -         Indicates the initiator shall send Default

Cumulative Values.

The following lists the possible save options for the LOG SELECT command.

All the Log Parameters which are selected for saving shall be saved to non-

volatile storage after the target performs the specified LOG SELECT

operation.

Further save operations are a function of the TSD bit in the Log Parameter

Control Byte.

Log Page

Parameter

CDB         Control

Values      Byte Value   Target Action

----------  ----------   -----------------------------------------------------

SP   PC     DS   LP

Bit  Field  Bit  Bit

---  -----  ---  ---

0    xx     x    x       Do not save any of the Log Parameters into non-

volatile storage.

1    00     0    0       Save all the Threshold Values of the selected Log

Page into non-volatile storage.

1    01     0    0       Save all the Cumulative Values of the selected Log

Page into non-volatile storage.

1    10     0    0       Save all the Default Threshold Values of the selected

Log Page into non-volatile storage.

1    11     0    0       Save all the Default Cumulative Values of the

selected Log Page into non-volatile storage.

1    xx     0    1       Save all the List Parameter Values of the selected

Log Page into non-volatile storage.

1    xx     1    x       Do not save any of the Log Parameters into non-

volatile storage.

The following lists the possible Parameter Values which can be controlled:

LOG        Log Page

SELECT     Parameter

CDB        Control

Values     Byte Value  Target Action

---------  ----------  -------------------------------------------------------

PC         LP

Field      Bit         Updated Parameter Value Usage

-----      ---         -------------------------------------------------------

00         0           The Parameter Values for all the Log Parameters in the

Log Page(s) sent to the target shall be used as

Threshold Values, unless the LP bit is set.

01         0           The Parameter Values for all the Log Parameters in the

Log Page(s) sent to the target shall be used as

Cumulative Values, unless the LP bit is set.

10         0           The target shall set the Current Threshold Values to

the Default Threshold Values for all the Log Parameters

specified in the Log Page(s) sent during a LOG SELECT

command, unless the LP bit is set.

11         0           The target shall set the Current Cumulative Values to

the Default Cumulative Values for all the Log

Parameters specified in the Log Page(s) sent during a

LOG SELECT command, unless the LP bit is set.

xx         1           Replace the Current List Parameter with the List

Parameter sent to the target.

Logging Operations

The logging operations can be setup to keep track of many different things.

What those things are is left up to the imagination of implementors.  But

what

should be well defined is how a target informs the initiator when a log

reaches a critical point thereby creating an exception condition.  What

follows is a description of how the target shall deal with these exception

conditions.

The following lists the definitions of the Parameter Control Byte of the Log

Parameter:

Parameter Control Byte values

------------------------------

DU   DS   TSD  ETC  TMC    LP

Bit  Bit  Bit  Bit  Field  Bit  Description

---  ---  ---  ---  -----  ---  ----------------------------------------------

0    -    -    -    --     -    Indicates that the target shall update the

log

parameter value to reflect all events that

should be noted by that log parameter.

1    -    -    -    --     -    Indicates that the target shall not update

the

log parameter value except in response to a

LOG SELECT command that specifies a new value

the log parameter.

-    0    -    -    --     -    Indicates that the target supports saving for

of the log parameter.

-    1    -    -    --     -    Indicates the target does not support saving

of the log parameter in response to a LOG

SELECT or LOG SENSE command.

-    -    0    -    --     -    Indicates that the target provides a target-

defined method of saving log parameters.

-    -    1    -    --     -    Indicates that either the target does not

provide a target-defined method for saving log

parameters or the target- defined method has

been disabled by the initiator.

-    -    -    0    --     -    Indicates that a comparison between the

threshold value and the cumulative value is

not performed.

-    -    -    1    --     -    Indicates that a comparison to the threshold

value is performed whenever the cumulative

value is updated

-    -    -    -    00     -    Indicates the target shall inform the

initiator on every update to the cumulative

value.

Parameter Control Byte values (Continued)

------------------------------

DU   DS   TSD  ETC  TMC    LP

Bit  Bit  Bit  Bit  Field  Bit  Description

---  ---  ---  ---  -----  ---  ----------------------------------------------

-    -    -    -    01     -    Indicates the target shall inform the

initiator every time the cumulative value is

equal to the threshold value.

-    -    -    -    10     -    Indicates the target shall inform the

initiator every time the cumulative value is

not equal to the threshold value.

-    -    -    -    11     -    Indicates the target shall inform the

initiator every time the cumulative value is

greater than the threshold value.

-    -    -    -    --     0    Indicates the log parameter is a data counter.

-    -    -    -    --     1    Indicates the log parameter is a list

parameter.

What follows is a description of how the target shall deal with exception

conditions.

Control

Log Page Parameter    Mode Page

Control Byte Values   (0Ah)      Target Action

--------------------  ---------  ---------------------------------------------

DU   ETC  TMC    LP   RLEC

Bit  Bit  Field  Bit  Bit        Exception Condition Actions

---  ---  -----  ---  ----       ---------------------------------------------

x    x    xx     x    0          No logging activities will cause a Contingent

Allegiance Condition or a Unit Attention

Condition.

x    0    GT 0   1    x          An illegal condition

x    1    xx     1    x          An illegal condition

0    1    xx     0    1          IF the threshold condition as defined

by the TMC field is met

a)IF there is an active I/O process

a)Complete the active I/O

process

b)If a Contingent Allegiance

Condition exists wait for it

to be cleared

END

b)Issue a Unit Attention Condition

to all Initiators

c)IF the Unit Attention Condition

is ignored

a)Continue normal operations

until the threshold condition

is met again

END

END

DU   ETC  TMC    LP   RLEC

Bit  Bit  Field  Bit  Bit        Exception Condition Actions

---  ---  -----  ---  ----       ---------------------------------------------

0    0    NV     0    1          IF a log counter reaches its maximum

value

1                                  a)Set DU to 1

b)IF there is no active I/O process

a)Wait until there is an

active I/O process

END

c)Complete the active I/O process

d)IF no Contingent Allegiance

Condition exists

a)Create a Contingent Allegiance

Condition with a sense key of

Recovered Error and an ASCQ of

Log Exception, Count At

Maximum

END

e)Wait for the Contingent Allegiance

Condition to be cleared

f)IF the cause of the counter

reaching maximum is not cleared

by the Initiator

a)Create a Contingent Allegiance

Condition every time the

counter should be incremented.

END

END

DU   ETC  TMC    LP   RLEC

Bit  Bit  Field  Bit  Bit        Exception Condition Actions

---  ---  -----  ---  ---        ---------------------------------------------

0    0    00     1    1          IF the log of parameters is full

a)Place the new log parameter code

value into the lowest parameter

code value position (wrap-around

the parameter codes)

b)IF there is no active I/O process

a)Wait until there is an active

I/O process

END

c)Complete the active I/O process

d)IF no Contingent Allegiance

Condition exists

a)Create a Contingent Allegiance

Condition with a sense key of

Recovered Error and an ASCQ of

Log Exception, List Codes

Exhausted

END

e)Wait for the Contingent

Allegiance Condition to be

cleared

f)IF the cause of the log of

parameters filling is not

cleared by the Initiator

a)Create a Contingent

Allegiance Condition every

time an entry is placed into

the log of parameters

END

END