X3T10/94-195

EXTERNAL MEMO

DATE:	October 27, 1994

TO:		X3T10

FROM:	Jim McGrath

SUBJ.:	IO Port List for IBM PC

In the search for unuded registers for ATA command queuing, I was told that some ATA tape drives are using address 03F3h. This is good input, although I would like some confirmation (and if this is the case, why has not anyone asked to reserve that byte in the ATA document?).  In the control block range, I see the following usage:

03F0    unused

03F1    unused

03F2    floppy

03F3    tape(?)

03F4    floppy

03F5    floppy

03F6    hard disk (bits 7-3 reserved)

03F7    floppy

Note that if worse came to worse we have bits 7-3 of an already allocated register, which gives us 5 bits or 32 tags.

I would like to suggest as working guidance to silicon people that they consider BOTH 3F3 and 3F6 (bits 7 through 3) to be possible for use  the ATA queuing protocol, and would like to echo John in requesting people to cite specific product conflicts - John is right, it will almost certainly be the case that queuing capability will have to be explicitly turned on by the host in any event, so a minor backward compatibility issue can be handled (some people don't like using 3F6 since it has the soft reset bit in it).

From "System BIOS for IBM PCs, Compatibles, and EISA Computers", 2nd edition, Phoenix Technologies, pp 84.

I/O addresses in the 3F2h-03F7h range are primary diskette controller addreses.  Bit settings also apply to addresses 0372h-0377h.

IO	Read/Write

Address	Status	Description

03F2h	W	Diskette controller digital output register, where:

bit 7	=	0	Reserved

bit 6	=	0	Reserved

bit 5	=	1	Drive 1 motor enable

bit 4	=	1	Drive 0 motor enable

bit 3	=	1 	Diskette DMA enable

bit 2	=	0	Controller reset

bit 1	=	0	Reserved

bit 0	=	0	Drive 0 select

=	1	Drive 1 select

03F4h	R	Diskette controller status register, where:

bit 7	=	1	Data register is ready

bit 6	=	1	Transfer is from controller to system

=	0	Transfer is from system to controller

bit 5	=	1	non-DMA mode

bit 4	=	1	diskette controller busy

bit 2-3	=	 	Reserved

bit 1	=	1	Drive 1 busy

bit 0	=	0	Drive 0 busy

03F5h	R/W	Diskette controller data register

03F6h	R	Fixed disk control port, where:

bit 7-4	=		Reserved

bit 3	=	0 	Reduce write current

=	1	Head 3 select enable

bit 2	=	1	Disk reset enable

=	0	Disk reset disable

bit 1	=	0	Disk initialization enable

=	1	Disk initialization disable

bit 0	=	0	Reserved

03F7h	R	Diskette digital input register, where:

bit 7	=	1	Diskette change

bit 6	=	1	Write gate

bit 5	=		Head select 3/reduced write current

bit 4	=		Head select 2

bit 3	=	 	Head select 1

bit 2	=		Head select 0

bit 1	=		Drive 1 select

bit 0	=		Drive 0 select

(bits 6-0 apply to the currently selected fixed disk drive)