To:		X3T10								X3T10/94-236r0

From:		Bob Snively

Sun Microsystems Computer Company

Mail Stop UMPK 12-204

2550 Garcia Ave,

CA 94043-1100

(415) 786-6694

Date:		November 22, 1994

Subject: 	Addressability of TARGET RESET task management function

The development of hierarchically organized address structures as described

in the SCC document (X3T10/1047D, Revision 3) requires the addition of a task

management function that can selectively perform a reset for a target below

the top level of the hierarchy.  To meet this requirement a LOGICAL UNIT

RESET task management function is proposed as an addition to the SAM document

(X3T10/994D, Revision 16).  This document proposes architectural concepts

and an implementation for the LOGICAL UNIT RESET function and proposes

wording additions to SAM to incorporate the new function.

Architectural Concepts:

At present, the TARGET RESET function acts on all logical units attached

to the SCSI port.  In the case of hierarchically organized address

structures, targets actually exist in the address space defined by the logical unit

number.  LOGICAL UNIT RESET is defined to perform the TARGET RESET function

on those targets and logical units existing in the LUN address space.  The

LOGICAL UNIT RESET function addressed to one of those targets or logical

units does not influence other peer units in the hierarchy and resets the

addressed unit and all targets and logical units below the addressed unit

in the hierarchy.

Proposed Implementation:

The object address for the LOGICAL UNIT RESET contains the logical unit

number of a destination logical unit.  The logical unit and all dependent

logical units will be reset.  SAM must be modified in the following manner.

Modifications to SAM:

Section 7 should have the following text added between the descriptions of

CLEAR TASK SET and TARGET RESET.

"LOGICAL UNIT RESET (Logical unit number||) - Reset the specified logical

unit and all targets and logical units hierarchically addressed from the

logical unit.  All tasks in all task sets for all affected logical units

are reset.  All hierarchically addressed SCSI devices shall support this

function.  For SCSI targets not having hierarchical addressing,

implementation of this function is a logical unit option."

The following new section 7.n is added between the present section 7.4 and

7.5:

7.n LOGICAL UNIT RESET

Function Call:

Service Response =

LOGICAL UNIT RESET (Logical unit identifier||)

Description:

This function shall be supported by all SCSI targets that support

hierarchical address structures.  The function is optionally supported by other SCSI

targets.

The logical unit addressed by the logical unit identifier is reset according

to the rules of the TARGET RESET function.  That logical unit and all lower

logical units in the hierarchy are reset.  Peer logical units and logical

units above the addressed logical unit are not affected by the

LOGICAL UNIT RESET function.

Before returning a Function Complete response the addressed logical unit

shall perform the hard reset functions specified in clause 6.6.6 and shall

create a unit attention condition in all affected logical units for all

initiators as specified in clause 6.6.5.