Fast-20 Comments and Implementors Note

There are a couple of items on the Fast-20 Spec that need to be

corrected..

In Section 6.1.1 the second paragraph conflicts with the first

paragraph and was previously agreed in the July meeting to

remove it and add an implementors note.

The second paragraph doesn’t make sense considering the cost of implementation on the high volume system. SCSI Plug and Play is

the only reference for termination outside of the devices. Fast-20

is an extension of SPI, the cost of integrating terminators into the drives, especially low end systems, is cheaper than adding extra connectors and external  termination.

The Regulated Terminator at the end of the cable currently isn’t

available with a source/sink regulator or just isn’t available completely.

Most of the plug on terminators are still passive. FAST 20 doesn’t allow

passive terminators.

The terminator integrated into the drive is typically a better

terminator than the ones used at the end of the cable. The

terminators at the ends of the cable are normally a regulator and

resistors. You should have a source/sink with active negation

drivers required by FAST-20. Standard regulators require external

components to make them sink current or an integrated terminator

with a source/sink regulator (Power Amplifier).

The typical cost of the terminator that plugs on the end of the

cable is $25-35. The connector is more expensive than the

terminators, the extra connector at the end of the cable, $2.50

plus the packaged terminator cost of $25-35. It cost $1.50 to $3.00 for

terminators integrated into an 8 bit device, even with a 2X

mark up it is not significant compared to the cost of a terminator

that plugs on the end of a cable. It would take an average of 5

devices on a SCSI bus to off set the cost. I believe I remember

a number like 3 or less as a typical SCSI bus number is 2 internal

and one external. The external termination is only a cost advantage on

servers and RAIDs that typically use more than 6 devices and

hot swap the devices.

It is a little harder to track termination, but this can be

worked.

I've received several comments about the cost of the packaged

terminator unit at the end of the cable from people on the

committee and customer. Unitrode make the integrated terminator,

but we do not make the package connector terminator. We sell to

the cable manufactures that make the package unit.

I would like to see the second paragraph in section 6.1.1

replaced with the following:

Terminators within a device must comply with all the device

guidelines. The termination enabled must only be at the end

of the SCSI bus, clear instructions shall be on the device of

disabling termination and enabling termination. The device total

capacitance must not exceed 25 pF. 15 pF has been achieved in

designs with disabled terminators on the device. Low capacitance

is recommended for better signal integrity. The terminator must

pass hot plugging test with or without TERMPWR.

Section 6.1.1 is still short one parameter. When several signal

lines are driven high with active negation drivers, if a Source/Sink

regulator is not used the reference voltage is pulled high. This

will cause the current required to pull down the low lines to

exceed 24 mA.

Item b should be worded:

b) Each terminator shall source current to the signal line

whenever its terminal voltage is below 2,5 V d.c. and this

current shall not exceed 24 mA for an line voltage above 0,2 V

d.c., even when the all other signal lines are driven to 4,0 V d.c.

Thank you,

Paul Aloisi

Unitrode