To:	Membership of X3T10

From:	Edward Lappin

Exabyte Corporation

tedl@exabyte.com

(303) 447-7718

Date:	November 15, 1994

Subject:	Determining the status of an immediate command

# Background:

During the SSC discussion on 11/10/94, it was concluded that we need a clarification on the definition of ready for sequential devices (such as tape).  We agreed on the following rules:

1.	 If the volume is being mounted or unmounted, the logical unit shall return CHECK CONDITION to a TUR command, with an appropriate status (depends if unloading or loading).

2.	If the volume is mounted, the logical unit shall return GOOD STATUS in response  to a TUR.

3.	If the volume is not mounted, the logical unit shall return NOT READY  in response to a TUR.

Therefore, the processing of an immediate command (such as erase immediate) will not change the ready status of the logical unit.  Because of this, another reporting mechanism is required to determine if an immediate command is in progress and the approximate stage of processing.

# Proposal:

1.	That a new ASC/ASCQ pair be added to SCSI-3 SPC.

| ASC | ASCQ | DTLPWRSOMC | Description |
| --- | --- | --- | --- |
| 00 | 16 | DTLPWRSOMC | IMMEDIATE COMMAND IN PROGRESS |

2.	That the Sense-key specific section of Request Sense in SCSI-3 SPC be amended to change the text for the progress indication bytes.

Old text:

If the sense key is NOT READY and the SKSV bit is one, the sense-key specific field shall be as shown in table 45.  These fields only apply to the FORMAT UNIT command with the Immed bit set to one.

New text:

If the sense key is NOT READY or NO SENSE and the SKSV bit is one, the sense-key specific field shall be as shown in table 45.

Old table caption:

Table 45 - Format progress indication bytes

New table caption:

Table 45 - Progress indication bytes

# Notes:

1.	This change removes the requirement that the FORMAT UNIT immediate command change the ready status to NOT READY.  If needed, this requirement can be stated in SBC.

2.	Any ASC/ASCQ may report a progress indication if the sense key is NOT READY.  However, to make sense, the progress indication should only be used if there is progress to report, such as progress towards ready, or possibly, progress towards completion of the not ready (such as unloading a tape).

3.	Any ASC/ASCQ may report a progress indication if2S2u2m2m2a2r2y2I2n2f2o2r2m2a2t2i2o2n222222222222222222222222222(22ÿÿÿÿÿÿÿÿÿÿÿÿ22222222222222222222222222222222222222222222222222222222222222222222222222222222222222222222222222222222222222222ÿÿÿÿÿÿÿÿÿÿÿÿ22222222222222222222222222222222222222222222222222222222222222222222222222222222222222222222222222222222222222222222ÿÿÿÿÿÿÿÿÿÿÿÿ22222222222222222222222222222222222222222222222222222222222222222222222222222222222222222222222222222222222222222222ÿÿÿÿÿÿÿÿÿÿÿÿ222222222222222222222222222222222222222222222222

333Edward Lappin333333333333333@333†
Cš”¹ 333333333333333333333333@333†—Èz¡¿¹ 333333333333333333333333@333†Ý‹ž¡¿¹ 333333333333333333333333333Z333333333333333333333333333333333›

333333333333333333333333333333@3333

3333333333333333333333333333-333333Microsoft Word 6.033333333333333333333333333333333333333333333-333333433333333333333333333333333333333333333333333333333333333333333ÐÏà¡±á3333333333333333;33þÿ	333333333333 333 3333333ÿÿÿÿÿÿÿÿÿÿÿÿÿÿÿÿÿÿÿÿÿÿÿÿÿÿÿÿÿÿÿÿÿÿÿÿÿÿÿÿÿÿÿÿÿÿÿÿÿÿÿÿÿÿÿÿÿÿÿÿÿÿÿÿ333´33|33ê33D

33d

33	33#	33¦33Ô33ë33ì33ð33ñ33ò33ó33÷33ø3333

3333	33

44

44<44ý4ý4û4ù4ý4ô4ô4ô4ô4ô4ô44ò444444444444444444444444444444444444444444