Michael Alexenko

Senior Design Engineer

2400 Trade Centre   Longmont, CO  80503

(303) 682-8378	(303) 682-8787(fax)

internet:  Mike.Alexenko@conner.com

This document addresses several issues relating to Revision 13 of the SPI, Annex B (normative), SCSI Configured Automatically (SCAM).

First, this document identifies and attempts to correct various typographical problems as well as identify areas that require clarification.  Next, this document requests changes to the specification, based on the experience of developing Plug and Play (SCAM) disk drives.  Lastly this document shares a few implementation observations.

This document does address issues that are outside of the scope of Annex B, and I apologize for that.  This was done primarily to use this forum as a means of sharing our experiences, and issues with other interested parties.  While these issues may fall outside of the scope of Annex B (at this time), it is our feeling that one of the purposes of a standards organization is to insure a usable and fully interoperable system that operates seamlessly and flawlessly to the user.  In striving to attain this goal we may need to reconsider the extent of Annex B’s definition, or create another Annex to address some of these other issues.

All SCAM technical references are from Revision 13 of the SPI, Annex B (normative), SCSI Configured Automatically (SCAM).

EDITORIAL ISSUES

1.	Section B.4.1.2, item #4 reads:

“Read and latch data from the DB(4-0) signals.”

This should read:

“Read and latch data from the DB(4-0) signals.  All devices assert the DB(6)

signal.”

(this is section B.4.1.1 in rev. 15)

2.	The note under figure 1 in Section B.4.1.2 reads:

“Note - Signals are shown”.

This should read:

“Note - Signals are shown asserted low.”

(this is figure B.5, section B.4.1.1 in rev. 15, and has been corrected)

3.	Section B.4.1.1 is confusing, and needs review.

In particular the references to the “device” requires some conscious interpretation.  In

this section “device” is interpreted as any SCSI electronic object that implements SCAM

and is capable of initiating the SCAM protocol.  I have pondered variations of this

section, and can only offer some suggestions, as opposed to a rewritten section:

a)	In paragraph one and two, “device” should be replaced with “SCAM

device”, as in paragraph three.

b)	Clarification in paragraph one that the protocol is initiated by either a

SCAM level 1 initiator, or a SCAM level 2 target, similar to what has

been done in paragraph two.

(this is section B.4.1 in rev. 15)

4.	Section B.4.1.1, paragraph two states “...After a variable delay, devices responding to

SCAM selection release the MSG signal.....SCAM targets should release the MSG

signal quickly, perhaps never asserting it at all. ....”

This section does not state when this signal is asserted by SCAM devices that have

responded to a SCAM selection.  This needs to be defined in a clear manner.

(this is section B.4.1 in rev. 15)

TECHNICAL ISSUES

1.	Section B.4.1.4 describes the ID string to be a maximum of 31 bytes, and that SCAM

initiators should be able to accept strings of 32 bytes, for future extensions.

It is assumed that this 32nd byte is appended to the Vendor Specific Code field (bytes

10-30), and there should be no reason why it can, or cannot be transmitted today (as a

zeroed byte).

Conner  requests that acceptance of the 32nd byte by SCAM initiators should

become mandatory, and this byte should be reserved (contents = 00h) at this time,

to avoid problems in the future with backward compatibility.

(this is section B.5.1.2 in rev. 15)

2.	Currently, it appears that no initiators are supporting the Configuration Process Complete

function code.  Section B.4.1.5.3 (section B.5.1.3 in rev. 15) defines this function

code (from a target perspective): "...A SCAM target with an unassigned ID that observes

this function code should not respond to selection until a reset condition, power on or the 	assignment of an ID during a subsequent SCAM protocol invocation."

Although section B.5.2 (section B.6.2, note 10, in rev. 15) includes an implementation

note: “Some SCAM targets do not recognize the Configuration Process Complete

function code and return to the SCAM Monitor state when SCAM protocol is

terminated.”, there are possible opportunities for problems by NOT supporting this

function code.

Conner requests that the implementation of this function code should become

mandatory for initiators and targets, to avoid problems such as ID conflicts with

fully loaded (and overloaded) busses.

It has been observed that in an illegally configured bus (one initiator and eight targets),

some initiators will assign all available IDs, and when detecting another device

requesting an ID, it will reset the bus and NOT retry the SCAM protocol, allowing ALL 	devices to use their default IDs.  Potentially all devices on the bus may have the same

ID, as defined in revision 1.0 of the “Plug and Play SCSI Specification”, dated March 30,

1994, depending on the device types used.

Granted, this may be beyond the scope of Annex B, and those initiators may be

behaving incorrectly, but mandatory implementation of the Configuration Process

Complete function code will help solve this and related issues.  It will also create a

mechanism for applications to provide added value to the end-user by correctly

identifying illegal bus configurations, and still allowing the bus to operate.

3.	Section B.5.1.1.1, paragraph two (section B.6.1.1 in rev. 15) , describes a situation where

the initiator is polling the bus to identify SCAM tolerant devices by attempting a

selection.  SCAM tolerant devices will respond by asserting BSY, and the initiator will

categorize that ID as assigned.  This section goes on to describe the next action by the

initiator: "...In this case, the dominant SCAM initiator should complete an Inquiry or

similar command sequence to gracefully conclude selection of the SCSI device...".

It has been observed that some initiators do not issue any command(s) to gracefully

conclude selection, but release signals, and\or issue resets.

It is Conner’s position that this section should be modified, and it should be made

mandatory for initiators to issue only an INQUIRY or TEST UNIT READY command

when polling the bus for legacy devices.  (Conner requests choosing one, and

not leaving it open to choice.)

4.	It has been observed that some initiators are not implementing Wired-OR Glitch Filtering

correctly (as defined in section B.4.1.3, or section B.5.1.1 in rev. 15)).  These initiators

will cease to participate in the SCAM protocol when DB7 is glitched.  Although it is the

responsibility of all initiators and targets to make every effort to not glitch the bus, all

targets and initiators have the added responsibility of properly deglitching the bus.

The following changes to this section are being requested by Conner:

a)	Delete the last paragraph regarding the “alternate method” of

calculating the iteration count.

b)	In item “2” require that the iteration count be 32, and delete

references to devices determining the width of the bus.  This will

insure a consistent and interoperable solution between vendors.

5.	Section B.4.1.5.1, paragraph 1 (section B.5.3.1 in rev. 15) states: "...At this point, the

SCAM initiator may transmit an action code to assign an ID to the device or perform an

additional function....".  (This is for the case where the initiator has transmitted the

Isolate function code (00000b).)

This may be interpreted as follows:

A device wins isolation, and the initiator then issues a Locate On action code.  At this

point the initiator would transmit another Synchronization function code, and all devices

would participate in another round of isolation. The same device would win, and wait for

the next action code.  The initiator would then transmit an Assign ID action code, and

this device would accept the new ID, and cease to participate in the SCAM protocol.

It has been observed that some SCAM targets will only accept an Assign ID action code

after winning isolation, meaning that if a Locate On action code is transmitted, it seems

to be discarded, and that device no longer participates in the SCAM protocol.  This

device will then respond only to its default ID, causing potential ID conflicts with legacy

devices

It is requested by Conner that the implementation of ALL function codes

described in section B.4.1.5,  and ALL action codes described in section B.4.1.5.1

become mandatory.  The table may look like this:

Function Codes

code					initiators	targets

isolate					M		M

isolate and set priority flag		O		M

configuration process complete	M		M

dominant initiator contention		M		---

Action Codes

code					initiators	targets

assign id 00nnnb			M		M

assign id 01nnnb			(depends)	(depends)

assign id 10nnnb			(depends)	(depends)

assign id 11nnnb			(depends)	(depends)

clear priority flag			O		M

locate on				O		M

locate off				O		O

(section B.5.1.3, table B.7, and  section B.5.1.3.1, table B.8, respectively, in rev. 15)

IMPLEMENTATION OBSERVATIONS

1.	Annex B does not define timeouts for any errors that may occur during the protocol, such

as:

- during SCAM Selection, any of the control lines or data lines used for

handshaking are never released by an initiator or target.

- the second quintet of an action code is never received.

- during the transfer cycles, a target or initiator never releases a handshake line

(DB7-DB5).

These issues should be examined, and appropriate timeouts and recovery actions should

be implemented.  A suggestion is to use the SCAM selection response time (250ms).

After this time has elapsed, the initiator(s) detecting the timeout should reset the bus.

This solution is obviously too simplistic, but may serve as a starting point for discussions.

Granted, an endless cycle of resetting the bus when the failing device continues to

“hang” is not desirable, but the current implementation of having no timeouts and

allowing the system to hang is not desirable either.  By defining some gross level of

timeouts, a means of allowing the application to be notified of an error is created.

2.	SCAM tolerant reset to selection delay is defined to be 250ms. This duration is defined

to be “The maximum time a SCAM tolerant device may delay after a reset condition

before enabling its response to selection”.  This is mentioned again in section B.5.1.1.1,

paragraph one (section B.6.1.1.1 in rev. 15): "After a reset condition, a dominant SCAM

initiator shall wait as necessary to insure that a SCAM tolerant reset to selection delay

has elapsed....".

The time from Reset to the polling of the bus to detect SCAM tolerant devices has been

measured on some host adapters to be in the range of 7-30ms.  This short duration

causes situations where some legacy devices do not respond to the initial Selection to

determine the bus configuration.  The result is potential ID conflicts with SCAM devices.

3.	The area of ID assignment for mixed bus widths is not specifically addressed.  Two

diagrams below describe such situations.  The first example is a 16-bit bus internally

(with devices that will support a 16-bit bus), and an 8-bit external bus.  The second

example describes a multi-initiator system with mixed bus widths.

SCAM Initiator

(ID 7)

8 - bit external bus

16 -bit

internal

bus

SCAM

Target

(ID ?)

SCAM

Target

(ID 6)

SCAM

Target

(ID 5)

{

ID 0 - 4?

or

ID 8 - F?

SCAM

Target

(ID ?)

Example 1 - Single Initiator, mixed bus widths.

Question: Do the devices on the 16-bit bus receive IDs greater than 07?

8 - bit external bus

Subordinate SCAM Initiator

(ID 6)

Dominant SCAM Initiator

(ID 7)

SCAM

Target

(ID 5)

8 - bit

internal

bus

16 - bit

internal

bus

SCAM

Target

(ID ?)

SCAM

Target

(ID 4)

{

ID 0 - 3?

or

ID 8 - F?

SCAM

Target

(ID ?)

Example 2 - Multiple Initiators, mixed bus widths.

Question: Do the devices on the 16-bit bus receive IDs greater than 07?

While perhaps beyond the scope of Annex B, it is Conner’s position that Annex B should make some reference to SCSI IDs in mixed bus width configurations.