To:		Membership of X3T10

From:		Edward Lappin

Exabyte Corporation

tedl@exabyte.com

Date:		January 19, 1995

Subject:	Addition of ASC/ASCQ codes for cleaning cartridges.

Background

For sequential devices, particularly tape, there is a need to determine when a cleaning cartridge is required.  This type of notification could be termed a maintenance request.

Additionally, the cleaning cartridge may be defective or completely used up.  This requires some notification as well.  Whether or not the cartridge is ejected after a cleaning attempt is device-specific.

Proposal

I am requesting that we add the following ASC/ASCQ pairs to SCSI-3 SPC.

| ASC | ASCQ | DTLPWRSOMC | Description |
| --- | --- | --- | --- |
| 00 | 17 | DT | CLEANING REQUESTED |
| 30 | 07 | DT | CLEANING CARTRIDGE FAILURE |