5/12/95						X3T10/95-242r0

TO:    X3 Secretariat

ATTN: Lynn Barra

1250 Eye Street NW, Suite 200

Washington, DC  20005

FROM:  Thomas Wicklund

Fujitsu Computer Products of America

1506 Harvard Street

Longmont, CO  80503

(303)-682-6549

FAX (303)-682-6401

email wicklund@intellistor.com

Public review comments of X3.276:199x, SCSI-3 Controller Commands (SCC)

I am working from document X3T10/1047D revision 4.

As a preliminary note, I use the RAID levels as defined in the

Berkeley papers.  I realize SCC doesn't explicitly define RAID levels

but it was easiest to describe common configurations.

General comments:

Interfaces used for devices:  There is an assumption throughout the

standard that the back-end interface of a SCSI-3 storage array is also

SCSI.  If this is intended as a requirement it should be stated up

front.  If not a statement should be made about non-SCSI back ends.

Types of devices: There is also an assumption that a SCSI-3 storage

array is an array of disk drives (or more generally SCSI block

devices).  This is not stated anyplace I can find but is implied by

the Normative Reference in clause 2 to SBC and the extensive use of

logical block addresses.  If devices must be block devices, this

should be stated, alternately a statement added that other device

types (e.g. sequential) may be used and the user can figure out a

reasonable way to map the device into the array.

The problem I see here would be an array device that also has the

ability to connect a tape backup or logging device.  There is no clear

way to configure a non-block device into a storage array.

Data mapping complexity:  The SCC standard provides a very flexible,

complex two level data mapping scheme.  One level maps physical blocks

to user or check data (converting a set of p_extents to a set of

ps_extents within a redundancy group), while the second maps data into

volume sets (ps_extents to volume sets).

Most of the parameters which specify the data mapping are specified

independently, allowing a different value for each unit being mapped

together.  However, in practice most RAID devices are going to define

relationships between these parameters.  If arbitrary parameters are

allowed (and not all combinations of parameters work), the LBA mapping

algorithm becomes extremely complex.

In addition, I don't see any way for a host to determine what

parameters a SACL supports, or what relationships are supported.

I think the complexity of the SCC standard is going to make it

unusable.  Creating a generalized array configuration utility for a

host will either involve a user manually entering the restrictions the

SACL requires or a lengthy trial and error process.

As a simple example for a single parameter, take the GRANULARITY OF

UNITS field which is used both when defining redundancy groups and

volume sets.  This field has values for bit, byte, 2-byte, 4-byte, and

logical block granularity (in addition to vendor specific).  I doubt

many real world array devices support all of these values (all I'm

aware of support only one or two values).  The implementer of a SACL

is left with a choice -- either accept any value and use what the

hardware supports (e.g. treat bit, byte, 2-byte, and 4-byte as the

single hardware sub-logical block mapping) or require that the host

keep trying values until one happens to work (which becomes much more

difficult if a RAID-3 device happens to use an 8-byte word, for

example, and to be honest should report Vendor Specific).

In redundancy groups, common RAID devices use the same NUMBER OF UNITS

OF CHECK DATA and NUMBER OF UNITS OF USER DATA fields for all

p_extents in a redundancy group.  In volume sets, the USER DATA STRIPE

DEPTH (specified individually per ps_extent) and NUMBER OF UNITS OF

USER DATA (specified individually in the underlying p_extents)

normally need a defined relationship between them or the mapping from

volume set LBA to p_extent LBA becomes convoluted.  Yet each parameter

is individually specified.

I think this complexity is going to leave SCC unusable.  I know it was

much harder to figure out than most standards.  I think a simpler

method of array configuration needs to be provided as an option or the

flexibility of SCC should be reduced so that the majority of products

don't need to deal with the complexity of checking all of redundant

SCC configuration parameters.

Related to the complexity issue, there is no way for a host to

determine the capabilities of an arbitrary SACL other than trial and

error.  Given the number of parameters which can be specified, it

isn't practical to produce a general array device driver, leaving one

little better than the current vendor specific solutions.

There is also no way to determine possible component device

attachments.  It appears a host must try to attach a component device

to a logical unit and see what happens.

Specific comments by page and clause:

Page 3, Clause 3.1.14, initiator:  Does this definition include SACL

devices?  The diagram on page 10 says an SACL can be an initiator, but

it doesn't really contain an application client.  The diagram on page

11 specifies the HBA as the initiator, but is the application client

limited to the HBA or does the SAM definition encompass the total host

system (HBA and above) as the application client?

Page 4, Clause 3.1.39, target:  A target is not restricted to a

SCSI-3 storage array device in this standard.  In fact, on page 11 the

diagram labels one box with "drive (target)".  This definition should

be expanded to include drives as targets.

Page 19, clause 5.2.1.4, Peripheral device address method:  The

second paragraph on page 19 starting "The TARGET/LUN field indicates

..." is not clear:

- If the BUS NUMBER is 0, the TARGET/LUN field refers to a LUN within

the current level of the storage array.  What is a LUN?  an SACL

defines volume sets, which are already addressable.  For SIP only, a

mapping is defined via a mode page to 5 bit LUN numbers.  I can't find

a definition of a LUN at the SACL level which makes sense here.

Page 22, clause 5.2.2.10, P_extent: Is this clause correct?  The

second paragraph states that "a single p_extent that contains no check

data may be configured into one or more redundancy groups."  Does this

mean that an assigned p_extent which contains no check data can be

configured into another redundancy group but if it contains no check

data it can't?

I think the paragraph should specify that an "unassigned p_extent may

be configured into one or more redundancy groups or one or more

spares".  As written, p_extents which contain check data may only be

configured into spares (since they can't be configured into redundancy

groups), which sounds wrong.

Should this clause contain a statement about how an application

client determines unassigned p_extents (analogous to the next clause

on ps_extents)?

Page 24, clause 5.2.2.12, Redundancy Group; also page 80-82, clause

6.4.1.2, Create/Modify Redundancy Group service action:

The NUMBER OF UNITS OF CHECK DATA and NUMBER OF UNITS OF USER DATA

fields are allowed to differ in each p_extent within a redundancy

group.  Are there real examples of RAID configurations which require

this?  The only example (in Annex C) uses the same value for each

p_extent, and in fact the example specifies the values once rather

than once per p_extent.

In the subclauses of this clause all of the standard data mappings

appear to require that NUMBER OF UNITS OF USER DATA be equal for all

p_extents, which implies that NUMBER OF UNITS OF CHECK DATA be equal

for all p_extents.  Vendor specific redundancy mapping may require a

different set of parameters (which SCC doesn't provide a means of

specifying).

Similarly there are specific requirements on the START CHECK DATA

INTERLEAVE UNIT for each device.  This field is either 0 for all

p_extents or must define a RAID-5 rotating parity scheme.

Looking further at each mapping defined by SCC, these parameters

aren't consistent in the mappings where they don't matter.  For the no

redundancy method, why is NUMBER OF UNITS OF USER DATA set to 0?  It

seems like it should be labeled as "set to desired value", whatever

that might be.

There is some justification to copy redundancy leaving NUMBER OF UNITS

OF CHECK DATA at 0, since each p_extent replicates the user data.

For XOR redundancy the NUMBER OF UNITS OF USER DATA must be equal for

all p_extents.  Yet for a traditional RAID-3 device I would think that

the check data p_extent would use NUMBER OF UNITS OF USER DATA equal

to zero and NUMBER OF UNITS OF CHECK DATA non-zero.  Otherwise the

host cannot specify which p_extent contains check data, which seems

odd since it specifies exactly where check data belongs in a RAID-5

type mapping.

If these parameters must remain as they are, an example of how

different values between p_extents are useful would help.

Page 25, Figure 12:  This flow chart only works for a RAID-5 type data

mapping.  If the parameters required for "no redundancy" or "copy

redundancy" mapping are plugged in the result is incorrect since the

first block of each p_extent becomes flagged as "check data" and the

"units of check data counter" decrements below zero, which doesn't

make sense in the context of the flow chart.  A similar problem occurs

in a RAID-3 type or XOR mapping.

Page 26, clause 5.2.2.12.3, XOR or P+Q redundancy...:  The standard

should define XOR and P+Q mapping.  Without a definition it appears a

product may use any check data mapping method it likes and call it

either XOR or P+Q, so long as the standard parameters are used to

define the mapping.

The RAID Advisory Board has defined these terms, but no RAB documents

are listed in the references to the SCC standard.

The RAB definition of P+Q mapping is two check blocks for N data

blocks allowing operation after 2 failures.  A P+Q mapping using

rotating parity with an odd number of p_extents cannot be specified

using the NUMBER OF UNITS OF CHECK DATA and NUMBER OF UNITS OF USER

DATA parameters.

Page 28, Figure 14:  An additional figure showing multiple volume sets

associated with one redundancy group will help illustrate that there

is no relationship between redundancy groups and volume sets since the

two are assigned independently.

Page 30 and 31, Figures 15 and 16:  This algorithm should include the

effect of setting the INCDEC field to 1 or state that it's a limited

example rather than a "general implementation".

Page 31, bottom of page:  The statement that "Figure 17 shows the most

general implementation of the three parameters used to map the user

data" is not true.  Figure 17 uses a constant "user data stripe depth"

while the Create/Modify Volume Set service action allows it to be

different for each ps_extent.

An example showing the utility of different "user data stripe depth"

values in each ps_extent should be added or this value should be made

constant for all ps_extents in a volume set.

Page 33, clause 5.2.3.4, Verify Action: This operation is more

properly handled by the existing VERIFY command.  If a volume set or

redundancy group uses a peripheral device type which doesn't define

the VERIFY command, then either VERIFY should be added to that

peripheral device type or the array verify operation is not

appropriate.

Page 35, Table 8:  First, this table is mis-labeled as applying to

disk storage devices.  Second, it seems like the RECEIVE DIAGNOSTIC

RESULTS command should be allowed, along with READ BUFFER and possibly

LOG SELECT / LOG SENSE.

Page 39, clause 6.1.1.1:  Under NUMBER OF BYTES PER LBA_P, I don't

think a value of 0 for a stream device makes sense since most current

stream devices use fixed block sizes.  This isn't required since

stream devices are already identified by the peripheral device type of

the assigned/unassigned p_extent descriptor.

Page 55, table 34, Logical Unit States Descriptor:  Why is the REPLACE

bit replicated on each state line?  Since this descriptor describes

one logical unit, it seems like it's either replaceable or not

replaceable.  Move the REPLACE bit to byte 1 or 4, or 5 of the

descriptor and allow the state to be an 8 bit value.  Otherwise

explain how a logical unit can have different REPLACE bit values for

different states returned.

Page 56, clause 6.1.1.7, Report States:  The last sentence of this

clause states that a target is operational if the READYING, ABNORMAL,

and NONAFAIL bits are 0.  If a p_extent fails and is rebuilt to a

spare, the ABNORMAL bit should be set to 1 (since an addressable

device has a state other than available.  This means a target is not

operational if any component (spare or not) has failed.  If this is

intended, a term other than "operational" should be chosen since most

users will consider an array which executes reads and writes to be

"operational" even if data needs to be rebuilt or a spare needs to be

replaced.

At the very least, define the term "operational".

Page 57, Table 36:  Move state Protection disabled into numeric order

with the rest of the codes or fix the table to be alphabetical.  It

appears an attempt was made to make tables 36-40 alphabetical but

tables 36 and 37 are not.

Capitalization should also be made consistent in the state names.

Page 57, Table 36, state "Exposed":  Change the description from "a

failure causes" to "a failure may cause".  A volume set can contain

multiple redundancy groups and only one may be exposed, while the

others are not.  Whether data is lost depends on which component

fails.

Page 57, Table 36, state "Readying", note 13: According to this note,

if the underlying drives of a volume set are not spinning (waiting for

a START UNIT) or otherwise not ready the volume set state is

"Readying", which seems to be inaccurate.  Either add a volume set

state for "Not Ready" (or perhaps "Present") or be sure the "Readying"

description clearly states that the volume set may not become

available without outside action.

Page 57, Table 36, states "Recalculate" and "Verify in progress":

Change these description to include the case where one or more

underlying redundancy groups is being recalculated or verified but not

the full volume set (analogous to the Rebuild state).

Page 58, table 37, state "Invalidated Protected Space":  Why not use

"Data lost" like volume sets?  Alternately, change the volume set

state to be "Invalidated Protected Space".

Page 58, table 37:  Shouldn't there be a "Spare in use" state as in

volume sets?  Without this state can a volume set easily detect the

"Spare in use" state by querying its redundancy groups?

Page 60, end of clause 6.1.1.7:  Move the last paragraph about the

replace bit to before all of the state tables.  At this point the

reader has forgotten about the structure being defined.

Page 62, top of page:  Reference the tables defining the different

DEVICE TYPE values.

Page 62, clause 6.2.1.2, "ATTACH TO COMPONENT DEVICE service action":

The first sentence in this clause refers to a logical unit in the

singular, but the service action can attach multiple logical units to

a component device.

Page 80, clause 6.4.1.1, ALLRG bit.  Also page 98, clause 6.6.1.1,

ALLVLU: The description of the ALLRG (ALLVLU) bit is wrong.  Zero or

one define whether the DISCHK bit applies to all redundancy groups or

the addressed redundancy group, the current definition conflicts with

DISCHK.

In addition, the description of DISCHK for both these clauses doesn't

define what happens if checking is disabled for a redundancy group

then enabled for the volume set containing the redundancy group.  Each

description states that after being disabled check data isn't

re-enabled until done so for the like structure (e.g. disable check

data on a redundancy group, must enable for a redundancy group).  As

stated the interaction is ambiguous.

Page 83, clause 6.4.1.2, Protected Space Pattern:   Shouldn't this be

a variable length field similar to the FORMAT command for disk drives?

I assume there was a good reason why a byte fill field wasn't used for

disks and it would seem to apply here also.

Page 84, clause 6.4.1.4, REBUILD P_EXTENT service action:  The first

sentence should include the possibility of rebuilding several

redundancy groups (both because of overlapping redundancy groups and

because a p_extent may consist of several redundancy groups).

Page 85, Table 73, Rebuild types:

A.  Why do codes 00b and 10b specify that assigned space be rebuilt

while for code 01b the full p_extent is apparently rebuilt whether

assigned or not?  Shouldn't code 01b also specify assigned space, or

perhaps text outside the table should say that this service action

"rebuilds the assigned space associated with the p_extent"?

B.  The second paragraph for each rebuild type (If the rebuild

operation fails...) should be removed from the table and made an

independent paragraph since it's identical for all rebuild codes.

C.  I think it would be better to require that the list of redundancy

groups be empty for codes 00b and 01b.  This is better than the

confusion which could result from ignored parameters.

Page 86, clause 6.4.1.5, REBUILD PERIPHERAL DEVICE service action:

The first sentence of this clause should include the fact that several

redundancy groups might be rebuilt.

Page 88, Table 76:  My earlier comments about table 73 also apply

here.

Page 98, clause 6.6.1.2, CONTROL WRITE OPERATIONS service action:

SCSI-2 already went through the question of whether to allow the host

to set write protect for a disk drive or not.  This service action is

more properly added to the SBC or similar standard as a write protect

capability.

This clause should also define whether rebuild and regenerate

operations are considered writes and whether a Modify or Delete action

is considered a write.

Page 100, clause 6.6.1.3, CREATE/MODIFY VOLUME SET service action:

The definitions at the bottom of page 100 (and also earlier in the

model) use the term "stripe".  It ought to be defined in the glossary.

At the bottom of page 100 it states that the PS_EXTENT STRIPE LENGTH

must be an exact multiple of the number of ps_extents.  I think this is

backwards, it should be that "the number of ps_extents is an exact

multiple of the PS_EXTENT STRIPE LENGTH".

Page 101, top of page:  The term "PS_EXTENT LENGTH field" is used.

I think "PS_EXTENT" should be lower case here (or it should be upper

case elsewhere in the text).  Looking back at table 65, I can't find a

"LENGTH" field associated with a ps_extent descriptor.  The "PS_EXTENT

LENGTH field" should be renamed to use the correct field name.

Page 101, PS_EXTENT INTERLEAVE DEPTH:  I can't figure out what to do

with this field from the text.  I can sort of figure it out from

figure 17, but it isn't clear what the units are.  Figure 20 makes

the use of this field apparent but the standard shouldn't depend on an

informative annex.

Page 101, Table 90:  This table is identical to table 84, should they

be combined?

Page 101, PS_EXTENT DESCRIPTOR, INCDEC field: Is this really

necessary?  I can't see any good reason why this feature is needed.

In fact, until disk drives have INCDEC fields setting this field to 1

would appear to be a guaranteed performance killer by trying to read

and write the drive backwards.

Also in this paragraph, it states "When INCDEC is one logical blocks

after the START LBA_PS field in the PS_EXTENT DESCRIPTOR shall be

assigned in descending order".  This seems to say assign the first

block to START LBA_PS, then assign the rest in descending order.

Page 103, clause 6.6.1.5, RECALCULATE VOLUME SET CHECK DATA service

action:  Why doesn't this service action have the three options

specified in the VERIFY RANGE of the next clause?  Minor

inconsistencies like this make using a standard interface very

frustrating since one is constantly annoyed that an option isn't

available for a particular operation and there's no obvious reason for

it.

Page 105, clause 6.6.1.6, VERIFY VOLUME SET CHECK DATA service action:

Why not redefine VERIFY RANGE so that bit 1 of byte 10 is an ALLVLU bit?

Then use bit 3 to specify whether the selected LBA range should be

used or not.

Page 106, first paragraph:  According to this paragraph, if a volume

set consists of 4 redundancy groups, and one redundancy type "no

redundancy", the other redundancy groups must be verified using a

VERIFY REDUNDANCY GROUP service action rather than the volume set

version.  Why not just state that redundancy groups with no redundancy

are ignored, especially if the LBA range being verified doesn't

include the "no redundancy" portion of the volume set.

Page 106, CONTVER bit:  What's the interaction between the CONTVER bit

in a volume set and the CONTVER bit of a redundancy group?  If

independent, it appears that a SACL must keep a CONTVER flag for each

ps_extent (since a redundancy group may have one setting while each

volume set using it may have another) and a bit for each redundancy

group the ps_extent is part of.

Page 123, Annex A:  The IDENTIFY message defined here uses bit 5 for

the VOLSEL bit.  The SIP standard specifies that bit 5 is reserved.

Either SCC should not use bit 5 (reduce the LUN size to 4 bits) or SIP

should be changed to remove the reserved bit from IDENTIFY.

Page 132, Figure 20:  What is a "strip depth" (I assume you mean

"stripe depth")?

This figure should state that an LBA_PS is 512 bytes (I think) and

that an LBA_V is 512 bytes (though there's no way to specify this

within SCC).