TO:  X3T10 Membership				Document X3T10/95-204, Revision 1

From: Bob.Snively@Eng.Sun.COM (Bob Snively)

To:	John Lohmeyer

Chairperson, X3T10

Symbios Logic Inc.

1635 Aeroplaza Drive

Colorado Springs, Colorado 80916

Subject:	Proposed response to public review comments on FCP

The accompanying pages document the proposed response to those public

review comments received about the draft proposed American National Standard

entitled Fibre Channel Protocol for SCSI, document X3T10/993D, Revision 10,

dated November 28, 1994.

The comments are identified by letters indicating the reviewer and by numbers

identifying the comments from each reviewer. The first section is a table

summarizing the comments and the response to each comment. The second

section is the text of the original comment and a formal response to the

comment, with accompanying explanations and justifications.

Sincerely,

Robert N. Snively, technical editor

Sun Microsystems

Mail Stop MPK 12-204

2550 Garcia Ave.

Mountain View, CA 94043-1100

phone:	415-786-6694

e-mail:	bob.snively@sun.com

Response to Public Review Comments

1)	Summary of responses:

The following table summarizes the public review comments received about the

dpANS

Fibre Channel Protocol for SCSI, document X3T10/993D, Revision 10, dated

November 28, 1994.

Comment	T/E	Title of Comment			Sections	Proposed

Changed		Resolution

HP 1	T	Provide Task Management Response	Many		Accept

HP 2	T	Target Reset effects PRLI				Withdrawn

HP 3	E	Clarify "Ambiguous Exchange"				Rejected

IBM 1	E	Section C.1, wording change		C.1		Accept

IBM 2	E	Section C.2, wording change		C.2		Accept

IBM 3	E	Section C.2, wording change		C.2		Accept

IBM 4	E	Section C, wording change		C		Accept

IBM 5	E	Section C.2, wording change		C.2		Accept

IBM 6	E	Section C.2, wording change		C.2		Accept

IBM 7	E	Section C.2, wording change		C.2		Accept

IBM 8	E	Annex C, wording change			C		Accept

IBM 9	E	Annex C, wording change			C		Accept

IBM 10	E	Section C.3, wording change		C.3		Accept

IBM 11	E	Section C.3.2, wording change		C.3.2		Accept

IBM 12	E	Section C.3.2, wording change		C.3.2		Accept

IBM 13	E	Section C.3.2, wording change		C.3.2		Accept

U 1	T	In Order Delivery			TBD		Accept

LLNL1	E	Comment on 2				2		Accepted

LLNL2	E	Comment on 3.1				2		Partial

LLNL3	E	Comment on 3.1.3 and 3.1.4		3.1.3/4		Accepted

LLNL4	E	Comment on 3.1.15			3.1.1.5		Accepted

LLNL5	E	Comment on 3.1.25			3.1.25		Accepted

LLNL6	T	Comment on 4.2				7.4.4,.5	Partial

LLNL7	E	Comment on 4.3				4.3		Accepted

LLNL8	E	Comment on 5.1				5.1		Accepted

LLNL9	E	Comment on 5.2, Table 6			5.2		Accepted

LLNL10	E	Comment on 5.5.6			5.5.6		Accepted

LLNL11	E	Comment on 5.5.11			5.5.11		Accepted

LLNL12	E	Comment on 6.1				6.1		Accepted

LLNL13	E	Comment on 6.2.2 - 6.2.4		6.2.2, .4	Accepted

LLNL14	E	Comment on 6.2.5			6.2.5		Accepted

LLNL15	T	Comment on 6.2.5			6.2.5		Accepted

LLNL16	E	Comment on 6.2.6.9 and 6.2.9.10		6.2.6.9		Accepted

LLNL17	E	Comment on 6.3				6.3		Accepted

LLNL18	T	Comment on 6.3				6.3		Accepted

LLNL19	E	Comment on 7.1.2.2			7.1.2.2		Accepted

LLNL20	E	Comment on 7.4						Rejected

LLNL21	E	Comment on C.2				C.2		Accepted

FSI 1	E	Add FQXID						Rejected

FSI 2	E	FQXID Definition					Rejected

FSI 3	E	Definition of Ports					Rejected

FSI 4	E	Unsolicited Command					Rejected

FSI 5	E	Starting Exchange					Rejected

FSI 6	E	Linking							Rejected

FSI 7	E	Task Management				TBD		Accepted

FSI 8	E	Task Management						Accepted

DEC 1	E	Section 3.1.5				3.1.5		Accepted

DEC 2	E	Section 3.1.7				3.1.7		Accepted

DEC 3	E	Section 3.1.9				3.1.9		Accepted

DEC 4	E	Section 3.1.10				3.1.10		Accepted

DEC 5	E	Section 3.1.12						Rejected

DEC 6	E	Section 3.1.13						Rejected

DEC 7	E	Section 3.1.15				3.1.15		Accepted

DEC 8	E	Section 3.1.18						Rejected

DEC 9	E	Section 3.1.20				3.1.20		Accepted

DEC 10	E	Section 3.1.25				3.1.25		Accepted

DEC 11	T	Definition of Tag			3.1.28		Partial

DEC 12	E	Section 3.1.33				3.1.33		Partial

DEC 13	E	Section 3.1.34				3.1.34		Accepted

DEC 14	T	Table 2					Table 2		Accepted

DEC 15	T	SAM Protocol Services					Rejected

DEC 16	T	SAM Protocol Specific Responses				Rejected

DEC 17	T	Asynchronous Event Reporting				Rejected

EMX 1	T	Annex A Extended Link Services				X3T11

EMX 2	E	Annex A Extended Link Services		A.2		Accepted

EMX 3	E	Annex A Extended Link Services		A.2		Accepted

EMX 4	E	Annex A Extended Link Services		A.3		Accepted

EMX 5	E	Section 6, PRLO/PRLI fields		6.2.6, .7	Accepted

EMX 6	I	Annex A Extended Link Services				Review

Text of public review comments and proposed response:

HP Section. Comments by Hewlett Packard, Kurt Chan

HP 1 	(Technical)		Provide Task Management Response

In the description of Task Management Functions on page 60 of the SCSI-3

Architecture

Model (SAM), service responses are required which indicate the status of the

function:

- Function Complete

-Function Rejected

-Service Delivery or Target Failure

SAM requires that one of the above service responses be returned for every

task management

function. However, FCP provides no means of providing that response. There

are valid reasons

why such a response is needed for multi-initiator, class 3 environments in

Fibre Channel

(examples provided upon request). It is inappropriate to rely on link layer

acknowledgments

(on class 1 or class 2 ACKs) to provide this service.

Therefore we suggest the following changes be made to FCP to make it

compliant with SAM,

and to provide a reliable, class of service-independent FC-4:

Table 5, Page 10

- Change T1 to: Command Request and Task Management Function

- Delete or Reserve T5. If this change is unacceptable for backwards

compatibility, add a PRLI

login option which specifies whether T1 or T5 is used to perform task

management (see next

page).

Table 6, Page 11

- Change I4 to: Command or Task Management Function Response.

7.1.2.2, Page 25

Change the first sentence to: "A task management function shall not be

requested in the same

Exchange as a SCSI Command."

7.4.5, Page 33

Modify RSP_CODE = 00 and add codes 04 and 05 to table 20:

RSP_CODE					Definition

00			No Failure or Task Management Function Complete

04			Task Management Function Not Supported

05			Task Management Function Failed

Insert the following text below table 20:

The task management function may or may not have been performed by the target

if

RSP_CODE is returned or if no FCP_RSP is returned before the Exchange is

aborted. Values

04 and 05 are not valid responses to SCSI commands.

B.1.9, page 53

Change the example to T1 instead of T5 and add the I4 response. Delete the

last sentence of the

first paragraph.

T1/I4 as a PRLI Option:

If a PRLI option is preferred for backward compatibility, add the following:

- Add note to table 5, page 10: "Using T1 to transfer Task Management

functions is only

allowable when enabled via PRLI."

- Add note to table 6, page 11: "Using I4 to transfer Task Management

functions is only

allowable when enabled via PRLI."

- Add as PRLI bit 7, word 3 to table 8, page 17: "Task Management Request

Response sent as

T1/I4." Add description of this PRLI option:

Setting this bit indicates a request for the SCSI Initiator to transmit Task

Management

Functions using T1, and for the SCSI Target to respond to Task Management

Functions using

I4.

The text then provides the behaviors selected if Word 3, Bit 7 is to be

defined.

HP 1:		Response to comment HP 1.

The comment is accepted. The PRLI option is not selected, so all

implementations are required

to make use of the T1/I4 mechanism for transmitting Task Management

Functions. No

modification to the PRLI text is required.

HP 2	(Technical)		 Target Reset effect on login parameters

Login parameters should be in the set of objects within a Target which are

restored to

their default conditions upon receipt of Target Reset.

After review, HP has withdrawn this comment.

HP 3	(Technical)		Clarify "Ambiguous Exchange"

The wording regarding "ambiguous exchanges" should be clarified by making

error

recovery within FCP profile-specific. Various profiles are explicit and

complete in

defining error recovery. The standard should only provide mechanisms by which

the

appropriate level of error recovery for the application can be assured. The

following

modifications should help clarify this.

Section 7.1.2.2

Attempts to define the term "ambiguous exchange" and the use of this concept

to invoke

error recovery is confusing and overly complex. Replace the last two

paragraphs of

TARGET RESET with:

The TARGET RESET is transmitted by the initiator (exchange originator) using

a new

exchange. Additional FC-PH recovery may be necessary following TARGET RESET.

Section 7.1.2.2

Attempts to define the term "ambiguous exchange" and the use of this concept

to invoke

error recovery is confusing and overly complex. Replace the last two

paragraphs of

CLEAR TASK SET with:

The CLEAR TASK SET is transmitted by the initiator (exchange   originator)

using a new

exchange. Additional FC-PH recovery may be   necessary following CLEAR TASK

SET.

Section 7.1.2.2

Attempts to define the term "ambiguous exchange" and the use of this concept

to invoke

error recovery is confusing and overly complex. Replace the second and third

paragraphs of ABORT TASK SET with:

The ABORT TASK SET is transmitted by the initiator (exchange   originator)

using a new

exchange. Additional FC-PH recovery may be   necessary following ABORT TASK

SET.

HP 3:		Response to comment HP 3.

The comment is rejected. For the operation of these task management functions

to be

sufficiently standard to operate correctly, the two ends of the link must

each reset any

command which may be in an unknown (ambiguous) state. Alternatively, all

outstanding commands must always be aborted by the initiator.

IBM Section. Comments by IBM, George Penokie

IBM 1 	(Editorial)		Section C.1, wording change

Page 58, Section C.1 - The statement "based on the SCSI committee's RAID

addressing

model" should read "based on the addressing model contained within the SCSI-3

Controller Commands Standard (X3T10/1047D)".

IBM 1: Response to comment IBM 1.

Accepted.

IBM 2 	(Editorial)		Section C.2, wording change

Page 58, Section C.2 - The section name should be changed to "Definition of

SCSI-3

Controller Commands (SCC) addressing model".

IBM 2: Response to comment IBM 2.

Accepted.

IBM 3 	(Editorial)		Section C.2, wording change

Page 58, Section C.2 - In the first sentence the statement "RAID addressing

model"

should be "SCC addressing model" and the statement "SCSI disk array (SDA)"

should

be "SCSI-3 storage array". In the last sentence the statement "of an SDA

logical unit,

all physical SCSI logical units, and volume logical units." should be "of an

SCSI-3

storage array, all peripheral device logical unit numbers, and all volume set

logical unit

numbers."

IBM 3: Response to comment IBM 3.

Accepted.

IBM 4 	(Editorial)		Section C, wording change

Rest of Annex C - All "SDA" should be changed to "SCSI-3 storage array".

IBM 4: Response to comment IBM 4.

Accepted.

IBM 5 	(Editorial)		Section C.2, wording change

Page 58, Section C.2, equations at end of page - The following should be

changed as

indicated:

P = Peripheral device

V = Volume Set

B = SCSI bus within the SCSI-3 Storage Array

IBM 5: Response to comment IBM 5.

Accepted.

IBM 6 	(Editorial)		Section C.2, wording change

Page 58, Section C.2, last sentence on page - The statement "address of a

Volume"

should be "address of a volume set".

IBM 6: Response to comment IBM 6.

Accepted.

IBM 7 	(Editorial)		Section C.2, wording change

Page 59, Section C.2, Table 44 - "Physical logical unit/SDA" should be

"Peripheral

device logical unit/SCSI-3 storage array" and "Volume logical unit" should be

"Volume

set logical unit."

IBM 7: Response to comment IBM 7.

Accepted.

IBM 8 	(Editorial)		Annex C, wording change

Rest of Annex C - All "RAID" should be changed to "SCSI-3 storage array".

IBM 8: Response to comment IBM 8.

Accepted.

IBM 9 	(Editorial)		Annex C, wording change

Rest of Annex C - All "physical device" should be changed to "peripheral

device".

IBM 9: Response to comment IBM 9.

Accepted.

IBM 10 	(Editorial)		Section C.3, wording change

Page 59, Section C.3, first sentence - The statement "logical volumes" should

be

"volume sets".

IBM 10: Response to comment IBM 10.

Accepted.

IBM 11 	(Editorial)		Section C.3.2, wording change

Page 60, Section C.3.2 - The title of this section should be "Addressing of

volume set

logical unit: example".

IBM 11: Response to comment IBM 11.

Accepted.

IBM 12 	(Editorial)		Section C.3.2, wording change

Page 60, Section C.3.2, first and second sentence - The statement "more disk

drive

Volumes. Volumes managed" should be "more volume sets. Volume sets managed".

IBM 12: Response to comment IBM 12.

Accepted.

IBM 13 	(Editorial)		Section C.3.2, wording change

Page 60 and 61, Section C.3.2 - The term "physical logical unit" should be

"peripheral

device logical unit" contained within the body of the standard. If it remains

it may

cause confusion.

IBM 13: Response to comment IBM 13.

Accepted.

Unisys Corporation Section, Comments by Jerry Witalka

U 1 	(Technical)		In Order Delivery

My current reading of the FCP specification requires all Initiators to

support an

FCP_XFER_RDY DATA_RO field that specifies out of order data delivery (SAM

random buffer access). I believe this is an unacceptable burden for those

Initiators that

are required to support unlimited scatter/gather operations. Our host

software is

allowed to specify scatter gather boundaries of a single main storage word.

Theoretically, we could be given a single 10000 byte disk read request that

could be

mapped to a 200 entry scatter/gather list specifying 200 different main

storage buffer

locations. If we had to support out of order data delivery, it could take an

unacceptable

amount of processing time to pore through the scatter/gather list and figure

out where

the starting storage address is for this Data IU as opposed to always

processing the

scatter/gather list sequentially.

Given this, I would like to request a change to FCP to allow an Initiator to

require in

order data delivery (SAM sequential buffer access) for all data transfers.

SIP has

effectively provided this functionality with the Enable Modify Data Pointer

(EMDP) bit

in the Disconnect/Reconnect mode page. The definition of this bit in the

SCSI-3

Primary Commands could be expanded to require the FCP_XFER_RDY DATA_RO

field specify in order data delivery or a new bit could be added to the

Process Login

FCP Service parameter page to require in order data delivery.

U 1: Response to comment U 1.

The committee recommends that the use of the Enable Modify Data Pointer bit

in the

Disconnect/Reconnect mode page be defined for FCP. Implementation of this bit

will

not be made mandatory in FCP for the following reasons.

1)	The architecture described in this comment does not make maximum use of

parallel

access disk systems that may return data most efficiently out of order. As a

result, the

performance of systems that require that all delivery of data be in order may

suffer.

2)	Class 2 and Class 3 data may be delivered out of order by some fabrics.

The fabric

login process that requests in order delivery on a frame by frame basis is

outside the definition

of FCP, but might be required for systems like those described.

Lawrence Livermore National Laboratories Section, Comments by Lansing Sloan

LLNL 001 (Editorial) Comment on 2

We believe FC-PH may be listed as a normative reference now (and deleted from

clause

8).

LLNL 001: Response to comment LLNL 001.

Accepted. (Verify with editors).

LLNL 002 (Editorial) Comment on 3.1

Many of the definitions include the text "[SAM]". Some have "[FC- PH]" or

"[FC-

AL]". Such text seems helpful but should the meanings should be explained, or

the text

deleted. Many of the terms with "[SAM]" are also defined in SAM Revision 016,

though not necessarily in SAM's "definitions" clause 4.1.

LLNL 002: Response to comment LLNL 002.

Partial. Section 2 explains how the references are generated and does

not specify where in the referenced document the referenced information

is found. The editor will use ANSI standard references. Brackets are

acceptable to avoid the repetitive wording: "As specified by..." Section 3

will get an additional indication that some of these are definitions from

other documents collected here for the convenience of the reader.

LLNL 003 (Editorial) Comment on 3.1.3 and 3.1.4

The terms "autosence buffer pointer" and "autosense returned flag" have

"[SAM]" in

their definitions in the FCP document but do not appear to be defined in SAM

Revision

016. (Both appear in SAM 012 clause 9.1 but not in SAM 016 clause 6.3.)

LLNL 003: Response to comment LLNL 003.

Since they are not used in the FC-4 document either, they should be deleted.

Accepted.

LLNL 004 (Editorial) Comment on 3.1.15

Since operation associators are 64 bits long, not 32, the FQXID with

operation

associators is a 176-bit concatenation, not 112.

LLNL 004: Response to comment LLNL 004.

Accepted.

LLNL 005 (Editorial) Comment on 3.1.25

The terms "SCSI command service" has "[SAM]" in its definition in the FCP

document

but does not appear to be defined in SAM Revision 016. (It is in SAM 012

clause 9.1

but not in SAM 016 clause 6.3, where it appears to have been replaced by

"Execute

Command" or "Send SCSI Command protocol service".)

LLNL 005: Response to comment LLNL 005.

The proper term is Execute Command. FCP will be changed accordingly.

Accepted. The

definition will have to be modified in a number of places in FCP.

LLNL 006 (Technical) Comment on 4.2, 4th paragraph, 2nd sentence

This sentence states that if an unusual condition has been detected then SCSI

REQUEST SENSE and FCP response information are returned. Is SCSI REQUEST

SENSE supposed to be returned even if Auto-sense is not specified? (If the

answer is

yes, that should be made explicit in FCP, since it sort of contradicts SAM.)

Similarly, if FCP response information is supposed to be returned regardless

of auto-

sense, that should be stated. (SAM presumably does not cover this.) Clauses

7.4, 7.4.5,

and/or 7.4.6 may be better places to clarify this.

LLNL 006: Response to comment LLNL 006.

Section 7.4.6 specifies that the proper FCP_SNS_INFO shall be presented when

the SCSI status byte of CHECK CONDITION or COMMAND TERMINATED is presented as

specified by SAM. It further indicates that FCP devices should always use

autosense.

Three additional issues have been identified while working with Mr. Sloane to

resolve this problem.

1) There is no specification defining when FCP Response Information must be

provided. Section 7.4.5 will be modified to indicate that FCP Response

Information

is provided when any of the RSP_CODE conditions are detected by the target.

2) There is a discrepancy in the definitions of the FCP_RSP_LEN field.

Section 7.4.4 will be modified to indicate that FCP allows a length of zero,

four, or eight when FCP Response Information is provided. Other lengths are

reserved for future standardization.

3) The document is not perfectly clear about whether or not the FCP_RSP_LEN

field is always provided, even if the FCP_RSP_LEN_VALID bit is set to zero.

The text of section 7.4.4 will be clarified to indicate that both length

fields

are always present, whether or not they are valid.

LLNL 007 (Editorial) Comment on 4.3, 2nd paragraph, 2nd sentence

The sentence says that task management functions... are always... the only IU

in a new

Exchange. According to 7.1.2.2, that's not true for "terminate task", which

appears to

be done only in existing Exchanges. Consider appending ", except for

Terminate Task"

to the end of the sentence.

LLNL 007: Response to comment LLNL 007.

Accepted.

LLNL 008 (Editorial) Comment on 5.1, 1st paragraph and Table 3

Please state clearly in the paragraph what the FCP_Port address identifiers

are. Please

use words that clearly say "D_ID" and "S_ID" in table 3 are the identifiers.

(The closest

words found in a quick scan are in the definitions of "target identifier" and

"initiator

identifier".)

LLNL 008: Response to comment LLNL 008.

These abbreviations and definitions will be added to sections 3.1 and 3.2.

LLNL 009 (Editorial) Comment on 5.2, Table 6

In the last line of the Note (and before the Key), delete the comma after

"I2". Also

delete "are usable to".

LLNL 009: Response to comment LLNL 009.

Accepted

LLNL 010 (Editorial) Comment on 5.5.6

Delete the "is" that precedes "identifies".

LLNL 010: Response to comment LLNL 010.

Accepted.

LLNL 011 (Editorial) Comment on 5.5.11, 2nd sentence

In "... Base Address is beginning address ...", insert "the" after "is".

LLNL 011: Response to comment LLNL 011.

Accepted.

LLNL 012 (Editorial) Comment on 6.1, second paragraph, second sentence

Change "separated processes" to "separate processes" following the third

instance of

"logically".

LLNL 012: Response to comment LLNL 012.

Accepted.

LLNL 013 (Editorial) Comment on 6.2.2 through 6.2.4

Each of these has words "for each FC-4" that I think are inappropriate in

FCP. Clause

6.2.4 has three instances of the phrase. The last paragraph before 6.1.1

certainly says

the parameters for the other FC-4's are outside the scope of FCP (properly).

Probably

the five instances of "for each FC-4" should be replaced by "for FCP".

LLNL 013: Response to comment LLNL 013.

The intent was to indicate that a PRLI could be performed for more than one

FC-4 at a

time. The best way to do this would be to indicate in 6.2 at the end of the

first paragraph

that image pairs for more than one FC-4 can be included in each PRLI command.

Then

the term "for each FC-4" could be entirely removed.

LLNL 014 (Editorial) Comment on 6.2.5, first sentence

Change "effects" to "affects."

LLNL 014: Response to comment LLNL 014.

Accepted.

LLNL 015 (Technical) Comment on 6.2.5, last paragraph, last sentence

Should "default" precede "PRLI" in: "... PRLI shall be present at the

completion of

PLOGI"?

LLNL 015: Response to comment LLNL 015.

The intent of this phrase is to indicate that implicit PRLI, if implemented,

is present at

the completion of PLOGI. The sentence will be changed to read:

If default PRLI information is complete enough so that N_Port login (PLOGI)

is sufficient to

perform an implicit PRLI, then PLOGI shall establish the same reset state and

Unit Attention

condition that would normally be established by PRLI.

LLNL 016 (Editorial) Comment on 6.2.6.9 and 6.2.6.10

In the first paragraph, fourth sentence, of each, a sentence starts with "If

either the

originator or the responder do not...". Change "do" to "does" in each clause.

LLNL 016: Response to comment LLNL 016.

Accepted.

LLNL 017 (Editorial) Comment on 6.3

In the second and third paragraphs, change "No further communication under

the

affected FC-4..." to "No further FCP communication...".

LLNL 017: Response to comment LLNL 017.

Accepted.

LLNL 018 (Technical) Comment on 6.3, next to last paragraph

The first sentence talks about "... the referenced process image...", and the

other two

sentences talk about a "PA". However, the content of this paragraph seems

equally

appropriate to communication between entities neither of which requires a PA.

If so, the

paragraph should be rewritten so that it does not seem to apply only when PAs

are used.

The best correction is unclear, but replacing "PA" with "image pair" may

help.

LLNL 018: Response to comment LLNL 018.

The first case shall be changed to refer to "communication with an image

that..."

The second case shall be changed to refer to "image.."

Accepted.

LLNL 019 (Editorial) Comment on 7.1.2.2, first paragraph

Replace the first sentence with something like: "Except for TERMINATE TASK, a

Task

management function shall be transmitted by the initiator (Exchange

Originator) using

a new Exchange. There is no response from the target for a Task Management

function."

LLNL 019: Response to comment LLNL 019.

Accepted.

LLNL 020 (Editorial) Comment on 7.4, first sentence

Insert "payload" after "IU" or in place of "IU".

LLNL 020: Response to comment LLNL 020.

IU is the correct term. Rejected.

LLNL 021 (Editorial) Comment on C.2

In the second line below Table 43, there should be one colon (not two)

following

"Generalized Address."

LLNL 021: Response to comment LLNL 021.

Accepted.

FSI Consulting Services Section, Comments by Gary Stephens

FSI 1 (Editorial) Add FQXID

Page 2, 3.1.15 Add the term "(FQXID)" behind the term name to be consistent

with

other places where the abbreviation appears behind the term.

FSI 1: Response to comment FSI 1.

Already contained in section 3.2, Abbreviations. The convention is, except

for the

glossary and the abbreviations, that the abbreviation is enclosed in

parentheses after the

defining words the first time the abbreviation is used in the text. Change is

not needed.

FSI 2 (Editorial) FQXID definition

Page 2, 2.1.15 In the process of a full Fibre Channel exchange, both the S_ID

and D_ID

fields may change. This optional function makes it difficult to specify a

fixed name for

the FQXID as specified in option b). Add some text to the standard to explain

the

migration of these values in a full Fibre Channel implementation. Such use is

not

prohibited by the standard and therefore must be described and mapped to FCP

constructs.

FSI 2: Response to comment FSI 2.

The conventions relating the S_ID and D_ID to the originator's N_Port ID and

the

responder's N_Port ID as a function of initiative are well defined by the

referenced

documents. There are no other valid changes in S_ID/D_ID values within an FCP

Exchange. This change is not required.

FSI 3 (Editorial) Definition of ports

Page 4, 4. 1, paragraph 4, line 2. The phrase "by a pair of N_Ports"' to "by

a pair of

FCP_Ports or a set of FCP_Ports using initial process associators". See

comment 2.

General The term N_Port should be removed from the document except in the

definition

of FCP_Port. The term N_Port does not include the definition of an NL_Port

for FC-

AL. The definition of FCP_Port should include the NL_Port for FC-AL.

FSI 3: Response to comment FSI 3.

After reviewing the text, the meaning described above appears to be included

correctly.

This change is not required.

FSI 4 (Editorial) Unsolicited Command

Page 5, Table 1. row 4 Remove the term Unsolicited in the FCP equivalent

column. The

command service request can be used for either an unsolicited command (i.e.,

the first

one for a new task) and also for subsequent linked commands which are

solicited by the

previous FCP_RESPONSE IU.

FSI 4: Response to comment FSI 4.

The term is correct as defined in FC-PH. The same category is used for both

the first

command and subsequent linked commands. No change is required.

FSI 5 (Editorial) Starting Exchange

Page 5, 4.2, paragraph 2 line 1 Change "starts an exchange by sending" to

"starts an

exchange for the first request by sending".

FSI 5: Response to comment FSI 5.

The statement appears to be correct. X_ID reassignment, which might appear to

be an

exception, is actually included in this statement, since reassignment of the

X_ID does

not terminate an exchange, but extends it under a different name. Command

linking,

which might appear to be an exception, is actually not part of this

statement, since this

indicates how an exchange begins. No change is required.

FSI 6 (Editorial) Linking

Page 5, 4.2, paragraph 2, line 3. Change "The FCP_CMND" to "The first

FCP_CMND".

Each FCP_CMND does not start an exchange in the case of linking.

FSI 6: Response to comment FSI 6.

The reference is correct as stated. The FCP I/O Operation for linked commands

starts

with the first command in the link and continues with subsequent commands. No

change

is required.

FSI 7 (Editorial) Task Management

Page 7, Table 2 Must be updated to match SAM functions for task management

along

with corresponding text additions.

FSI 7: Response to comment FSI 7.

Accepted. The task management definitions in SAM will be reviewed and any new

information or changes will be reflected if required in FCP.

FSI 8 (Editorial) Task Management

Page 10, 5.3, Table 5 Adjust the T1 IU to include confirmed task management.

FSI 8: Response to comment FSI 8.

Accepted, See HP 1

Digital Equipment Corporation Section, Comments by Charles Monia

DEC 1 (Editorial) Section 3.1.5

All text after the first sentence is explanatory material which

is outside the scope of the definition and should be deleted.

DEC 1: Response to comment DEC 1.

Accepted.

DEC 2(Editorial) Section 3.1.7

See comment 001.

DEC 2: Response to comment DEC 2.

Accepted.

DEC 3 (Editorial) Section 3.1.9

See comment 001

DEC 3: Response to comment DEC 3.

Accepted.

DEC 4 (Editorial) Section 3.1.10

See comment 001.

DEC 4: Response to comment DEC 4.

Accepted.

DEC 5 (Editorial) Section 3.1.12

See comment 001.

DEC 5: Response to comment DEC 5.

This section summarizes the definition of a very bulky section of FC-PH. This

information is important to an understanding of the choices and definitions

made in

FCP, so I recommend that it remain unchanged.

DEC 6 (Editorial) Section 3.1.13

See comment 001.

DEC 6: Response to comment DEC 6.

This clarification is an appropriate part of the definition because it

relates SCSI and

FC-PH concepts, so I recommend it remain unchanged.

DEC 7 (Editorial) Section 3.1.15

See comment 001

DEC 7: Response to comment DEC 1.

Accepted.

DEC 8 (Editorial) Section 3.1.18

See comment 001.

DEC 8: Response to comment DEC 8.

This clarification is an appropriate part of the definition because it

relates SCSI and

FC-PH concepts, so I recommend it remain unchanged.

DEC 9 (Editorial) Section 3.1.20

See comment 001.

DEC 9: Response to comment DEC 9.

Accepted.

DEC 10 (Editorial) Section 3.1.25

See comment 001.

DEC 10: Response to comment DEC 10.

Accepted

DEC 11 (Technical) Definition of tag

Section 3.1.28

The definition for "tag" in the first sentence should be changed

to: "The initiator-specified component of the task identifier."

All text following the first sentence should be deleted.

DEC 11: Response to comment DEC 11.

The first sentence should be changed as requested to match up with SAM. The

clarifying sentence beyond that is an appropriate part of the definition

because it relates

SCSI and FC-PH concepts, so I recommend it remain unchanged.

DEC 12 (Editorial) Section 3.1.33

See comment 001.

DEC 12: Response to comment DEC 12.

The second sentence only will be deleted. The third sentence is an

appropriate part of

the definition because it relates SCSI and FC-PH concepts, so I recommend it

remain

unchanged.

DEC 13 (Editorial) Section 3.1.34

See comment 001.

DEC 13: Response to comment DEC 13.

Accepted

DEC 14 (Technical) Table 2

The names in parentheses in column 1 appear to be the names of

SIP messages corresponding to the specified task management

functions. These names should be deleted.

DEC 14: Response to comment DEC 14.

Accepted.

DEC 15 (Technical) SAM Protocol Services

In clauses 6.3 and 7.7 of SAM, protocol services to be provided

by an LLP are defined that support the command and task

management functions. The FCP protocol description should

reference the service and it's arguments by name. I would expect

to see a clause in FCP for each applicable SAM task management

or command protocol service. In addition to showing the service

interface and its parameters, the clause should map each SAM

parameter into its FCP-equivalent and describe corresponding FCP

behavior.

DEC 15: Response to comment DEC 15.

This is not really a technical comment, but an editorial. The functionality

required by

SAM is included in FCP, but is not organized to explicitly relate the

required services to

the FCP implementation of those services. In some cases using Class 3

services and

certain profiles, the SAM service is provided through a "Negative

Acknowledgment".

In those cases the proper behaviors are assumed until a time-out or other

error

indication indicates that the behavior was not complete. The FCP should not

be

rewritten to match the SAM format until FCP-2 is published.

DEC 16 (Technical) SAM Protocol Specific Responses

As required by clauses 6 and 7 of SAM, FCP should identify the

protocol-specific responses corresponding to the following

service responses:

Task Complete, Linked Command Complete, Linked Command Complete

(with flag), Service Delivery or Target Failure, Function

Complete.

DEC 16: Response to comment DEC 16.

This is not really a technical comment, but an editorial. The functions

required by SAM

are described by FCP, but not organized to explicitly relate the SAM

responses to the

FCP implementation of those responses. In particular, the Task Complete

information is

provided by the FCP_RSP. Linked Command Complete is provided by the same

FCP_RSP indication as task complete, but with different IUs. Linked Command

Complete with Flag is distinguished from Linked Command Complete by state

information retained by the host adapter. While these editorial changes would

be nice,

the FCP should not be rewritten to match the SAM format until FCP-2 is

published.

DEC 17 (Technical) Asynchronous Event Reporting

The mechanism for Asynchronous Event Reporting is not defined as

required by clause 6.6.4.1 of SAM.

DEC 17: Response to comment DEC 17.

Asynchronous Event Reporting is defined as a SEND command from a device that

normally acts as a target, but is temporarily acting as an initiator. This is

explained at

the end of section 4.2. No changes are required.

Emulex Section, Comments by Greg Scherer

EMX 1 (Technical) Annex A (normative) Extended link services

Background for the comment:

In Annex A of FCP an FC-4 login/logout service is defined in order to

implement specific FCP functionality. This login/logout service is stated

to be generic and therefore designed to support multiple FC-4's within it's

framework.

The login service (PRLI) supports FC-4 TYPE and or Process Associators to be

exchanged during login, in order to differentiate between separate FC-4

logins

(e.g. FCP, IPI, etc.), or even separate FC-4 image pairs (FCP image pair 1,

FCP image pair 2, etc.). This architecturally allows two N_PORT's with

independent FC-4's to communicate using independent FC-4 login parameters.

My issue comes from the fact that the logout service (PRLO) does not support

FC-4 TYPE specific logout. This means that if two N_PORT's choose not to use

Process Associators, (that will ensure image pair uniqueness), any PRLO

(logout)

from a single FC-4 will affect all others. Although today there may not be

many

coexisting FC-4's that use PRLI/PRLO, I believe the intent in Annex A was to

document an enabling service that could be used in a much broader scope in

the future.

Specific comment:

In Annex A the PRLO / ACC payloads define Parameter Page Word 0 Bit 31:16 as

RESERVED.   This is the same field in the PRLI / ACC payload that is defined

as the FC-4 TYPE and TYPE code expansion. Without the TYPE and TYPE code

expansion area defined in the PRLO / ACC payload it is not possible (without

using Process Associators) to logout one FC-4 on a given N_PORT without

potentially affecting others.

Recommendation:

Add FC-4 TYPE and TYPE code expansion fields to the PRLO / ACC payloads, as

per the definition and field position of TYPE and TYPE code expansion fields

defined in the PRLI / ACC payloads.

In any case, tables 11 and 12 (PRLO / ACC payloads listed in section 6) must

be made to match tables 29 and 31 (from Annex A) as they currently contradict

each other regarding the definition of Parameter Page Word 0 Bit 31:16 (TYPE

and TYPE code expansion fields).

EMX 1: Response to comment EMX 1.

Since the actual standardization of this function is being carried forward

as an FC-PH-2 activity, this will ultimately be a comment against that

document.

The editor will review FC-PH-2 for conformity and make those corrections that

are presently defined. This is a desirable characteristic which should be

carried forward.

The contradiction will be resolved. This comment is accepted in principle,

but

requires supporting activities from another organization to be completed.

EMX 2 (Editorial) Annex A (normative) Extended link services

Page 39 first bullet:

- Word 0, Bit 14 Establish Image Pair

Should be bit 13 (as listed in table 24).

EMX 2: Response to comment EMX 2.

Accepted.

EMX 3 (Editorial) Annex A (normative) Extended link services

Page 41 fourth bullet:

- Word 0, Bit 14 Establish Image Pair

Image pair established only if bit 14.......

Should be bit 13 (as listed in table 26).

EMX 3: Response to comment EMX 3.

Accepted.

EMX 4 (Editorial) Annex A (normative) Extended link services

Table 31 entry 7 is 13-0 should be 7-0:

Item 					Word 		Bit

------------------------------------------------------------------

Logout parm response page 		0-3 		31- 0

Reserved 				0 		31-16

Originator PA Valid			0 		15

Responder PA Valid			0 		14

Reserved 				0 		13-12

Response code 			0 		11- 8

Reserved 				0 		13- 0 <------

EMX 4: Response to comment EMX 4.

Accepted.

EMX 5 (Editorial) Section 6 PRLO / PRLI Field definitions

Tables 8 and 10 document Parameter Page payload Word 0 bit 13 as

RESERVED. This same bit in tables 24 and 26 (of Annex A) is defined

as "Establish Image Pair" an "Image Pair Established" respectively.

The bit definitions in both sets of tables (8 / 24 and 10 / 26) and

text should match.

EMX 5: Response to comment EMX 5.

Accepted. Section 6 will be updated.

EMX 6 (Improvement) Annex A (normative) Extended link services

I would like to see more detail in the general description for PRLI

Parameter Page Word 0, bit 13 "Establish Image Pair". If this bit = 1

(Establish Image Pair and exchange parameters), must bits 15-14

(Process Associators) also be =1?   There is no indication in the

text that they are tightly coupled, although I believe that they

are. Relationships such as these should be clearly defined (even

though potentially obvious).

EMX 6: Response to comment EMX 6.

This revision did not require Process Associators to establish an

image pair. When the PA is invalid, then the image is of the whole

node. This was kept vague so that either mechanism could be used.

The editor will review the text with respect to the latest revision

of FC-PH-2.