X3T10/95-205R0

2190 Miller Drive · Longmont, Colorado 80501-6744 · (303) 678-2149 · Fax (303) 678-2165

To:	ATA-3 Working Group

From:	Pete McLean

Date:	4 May 1995

Subj:	ATA-3/ATAPI differences

The following are the items that I found where the ATAPI document redefines or adds to the definitions in the ATA-3 document. This list may not be complete, these are simply the ones that I noted. We need to decide whether we will ignore this issue, reference the ATAPI document where applicable, or add the ATAPI definitions to the ATA-3 document.

Clause 5 - Signals

- Behavior of PDIAG- and INTRQ signals is different for ATAPI commands (“strong overlap”).

- Response to the RESET signal is different in ATAPI.

Clause 6 - Registers

- Contents of the Features, Cylinder Low, Cylinder High, Error, Sector Count, and Sector Number registers are redefined for ATAPI commands.

- Definition and/or behavior of the DRQ, DSC, DRDY, and BSY bits in the Status and Alternate Status registers different for ATAPI commands.

- SRST bit in the Device Control register not used in ATAPI.

Clause 7 - Reset responses

- The Power-up, RESET signal, SRST bit responses are different in ATAPI.

Clause 8 - Commands

- ATAPI defines four new commands with new command codes.

- Protocol for some of the new commands differs from any currently defined command protocol.

- New commands redefine register contents when issued.

- List of mandatory/optional commands for ATAPI is different from ATA-3.

- Commands using the Standby Timer are forbidden in ATAPI.

Clause 9 - Protocols

- Protocol for ATAPI commands is different from any current ATA-3 protocol.