To:		Membership of X3T10

From:		Arlan Stone

Unisys Corporation

arlan.stone@mv.unisys.com

(714) 380-5982

Date:		September 14, 1995

Subject:	Log page and parameters for SCSI-3 SSC   X3T10/95-304R1.

I am requesting that a new log page and log parameters for sequential-access devices, be added to SCSI-3 Stream Device Command Set (SSC).  They will assist customer service by:

1.  Allowing the client application to determine whether requested cleaning of the tape drive head is done promptly, or at all, by the end user.  This information is useful when complaints about drive reliability and error rates are being fielded by customer service.

2,  Allowing the client application to determine the compression ratio.  This information is useful when helping the customer understand the actual, versus perceived, versus expected data compression.

Changes to Subclause 5.3.2 Log parameters

Add the following to Table 22:

Page Code:  0Ch

Description:  Sequential-access device page

Subclause:  5.3.2.1

Added Subclause 5.3.2.1 Log parameters

The sequential-access device page (page code 0Ch) defines data counters associated with data bytes transferred to and from the media and to and from the initiator, and a list parameter of binary information on cleaning.

Support of this page and parameters are optional.

Table XX defines the parameter codes for the sequential-access device page.  The default value of all parameters shall be zero.  A non-zero value of the cleaning required parameter indicates that a condition requiring cleaning has been detected and a subsequent cleaning cycle has not been completed.  Cleaning required shall be persistent across hard resets and power cycles

Table XX - Parameter codes for sequential-access device page

| Parameter code | Description |  |
| --- | --- | --- |
| 0000h 0001h 0002h 0003h 0004h-00FFh 0100h 0101h-7FFFh 8000h-FFFFh |  | Number of data bytes received from the initiator during WRITE command operations Number of data bytes written to the media as a result of WRITE command operations, not counting ECC and formatting overhead Number of data bytes read from the media during READ command operations, not counting ECC and formatting overhead Number of data bytes sent to the initiator during READ command operations Reserved Cleaning required Reserved Vendor-specific |