Doc.No. 	X3T10/95-305r0

Date:	              Aug 7, 1995

Project:	X3T10-1048D

Ref. Doc.:	X3T10/94-057

Reply to:  	R.K. Roberts

To:	Membership of X3T10

From:	RK Roberts

Subject:	 Minutes of X3T10 MMC  Working Group - Aug 2 & 3, 1995

The meetings were held in Sunnyvale, CA  hosted by Philips Semiconductor.

1.	Opening Remarks

Ron Roberts convened the meeting at 9:45 AM on Aug 2, 1995.  He thanked our

host, Kevin Ross and Philips Semiconductor, for hosting the meeting.  It was stated that the meeting was authorized by X3T10 and will be conducted under the X3 rules.  Ad hoc meetings take no final actions, but prepare recommendations for approval by the X3T10 technical group.  The voting rules for the meeting are those of the parent committee, X3T10.  For the ad hoc, other than straw votes, the voting rules  are:  one vote per participating company.

It was also stated that the minutes of this meeting will be posted to the X3T10 BBS, the SCSI Reflector, the MMC Reflector, the ATAPI Reflector, and will be included in the next X3T10 committee mailing. Attendance at working group meetings does not count toward minimum attendance requirements for X3T10 membership.  Working group meetings are open to any person or company to attend and to express their opinion on the subjects being discussed.

2.0 	Introductions & Attendance

As is customary, the people attending introduced themselves.  An attendance list  was circulated for attendance and corrections to addresses and e-mail.  The list of attendees is attached.

3.0 	Document Distribution

The minutes from July meeting was handed out. Also a copy of the current Functional  Requirements (Rev 2) was made available.

---------------------

95-223R2	MMC Functional Requirements

95-286R0	MMC Working Group minutes  July 1995

4.   Call for Patents

Ron requested that any patents known to the working group be identified.

5.  Approval of Agenda

The agenda was approved with two additions. Added 6.2.2. SMC command review and 7.2 CD-E Technology

6.0 	Meeting Agenda

6.1	Functional Requirements Doc

Coverred the remaining action items from the two meetings ago. See open action lists..

1.) Mike Yokoyama of SONY ... Presented Track at Once, in response to action item of June WG session,  requirements. His presentation follows:

1. Track Status

The following tracks accept TAO recording:

1)  invisible Track ( Blank Area)

2)  Empty Reserved Track indicated vby the PMA item of CONTROL 3D 0x4 (TAO)

2. Required Parameteres

Case 1:  Blank disc (invisible Track & the 1st track in the disc)

Disc Type

First Track Number

Data Format  (mode-1, mode-2,mode-2 form-1, mode-2 form-2,audio)

Copy bit  ( Copy Inhibited, Copy Permitted, Replica)

TDB Type

Block Length

Track Length

Case 2:  Invisible Track & not the 1st track

Track Number

Data Format  (mode-1, mode-2,mode-2 form-1, mode-2 form-2,audio)

Copy bit  ( Copy Inhibited, Copy Permitted, Replica)

TDB Type

Block Length

Case 3: Empty Reserved Track

Track Number

Data Format  (mode-1, mode-2,mode-2 form-1, mode-2 form-2,audio)

Copy bit  ( Copy Inhibited, Copy Permitted, Replica)

TDB Type

Block Length

Case 4: Empty Reserved Track & not the 1st track in the program

Track Number

Data Format  (mode-1, mode-2,mode-2 form-1, mode-2 form-2,audio)

Copy bit  ( Copy Inhibited, Copy Permitted, Replica)

Block Length

TDB Type-1  =3D  all previous track information included

TDB Type-2  =3D  only current track information

3. Drive Action

Case 1: Blank disc (invisible Track & the 1st track in the disc)

* Drive checks Data Format is appropriate to Disc Type

* Drive checks Block Length is appropriate to Data Format.

* Drive prepares and writes Track Descriptor blocks (pre-gap) with

the format specified by the Data Format and Copy Bit.

* Drive receives and encodes the data of the track.

* After writing the data of the length specified by Track Length, Drive ends writing.

* Drive writes the item of the track in PMA.

Case 2: Invisible Track & not the 1st track

* Drive checks the status of the track specified by Track Number

* Drive checks Data Format is appropriate to Disc Type

* Drive checks Block Length is appropriate to Data Format.

* Drive prepares and writes Track Descriptor blocks (pre-gap) with

the format specified by the Data Format and Copy Bit.

* Drive receives and encodes the data of the track.

* After writing the data of the length specified by Track Length, Drive ends writing.

* Drive writes the item of the track in PMA.

Case 3: Empty Reserved Track

* Drive checks the status of the track specified by Track Number

* Drive checks Data Format is appropriate to Disc Type

* Drive checks Block Length is appropriate to Data Format.

* Drive prepares and writes Track Descriptor blocks (pre-gap) with

the format specified by the Data Format and Copy Bit.

* Drive receives and encodes the data of the track.

* After writing the data of the length specified by Track Length, Drive ends writing.

* When writing reaches the end of the track, Drive ends writing.

Case 4: Empty Reserved Track & not the 1st track in the program

* Drive checks the status of the track specified by Track Number

* Drive checks the item of the track in PMA.

* Drive checks Block Length is appropriate to Data Format.

* Drive prepares and writes Track Descriptor blocks (pre-gap) with

the format that is the same as the previous track=92s except the Copy bit.

* Drive receives and encodes the data of the track.

* When writing reaches the end of the track, Drive ends writing.

---------------------------------------------------------------------------------------------------------------------

2.) Ron led discussion on Complete packet Track primitives.  Flows were modified, see new  Figure 9 in document 95-223-R3.  Primitives developed are attached below 2E

1) PARAMETERS

a)  Track Number

b)  Track Variables

c)   Last Link Position

d)  What is the =93Pad Data=94

2)  PRIMITIVE ASSUMPTIONS

a)  NOT  Track at Once

b)  NOT Session at Once

3)  QUERY PRIMITIVES

a)  Is track entry in PMA?

yes - track is reserved

no - incomplete or invisible

b)  If reserved

what is last link position?

Is track filled?

4)  PRIMITIVES:

- Reading PMA looking for track number

- Read TDB to get Track Varibles

- Get Last Link Position

- Write to PMA

- Write Packet ( Fixed or Variable) using PAD data

- Write TAO using PAD data

---------------------------------------------------------------------------------

During discussions, the group determined the need for a =91Disc Information=92 function with the following:

Information Required		How  toget it

----------------------------------------	-------------------------------

Disc Style uninterrupted ROM	test PMA/TOC/0:0:0

interrupted		CD-R

Disc State  (empty)		created from sec 5.5 in Funct. Req. Doc.

Disc Type Audio/ROM/XA/CD-I.	Read TOC

Appendable			Read TOC,  B0 pointer

First  Track Number		Read PMA/TOC

Last  Track  number		PMA or PMA+1 if Read PMA/TOC unless track includes appendable incomplete track

Number of Sessions		PMA and TOC(s)

Disc Application Code		Read TOC comes from ATIP Restricted/unrestricted

Recording Power			Read ATIP

Start of LI - USER AREA- LO	Read ATIP

Last Session Status 		(empty - partially Read pre-gap of last Track on last session written)

UPC				TOC or new

--------------------------------------------------------------------------------------------------------------

The WG then determined that a =93Track Information=94 function was also required.

TRACK INFORMATION

Information Required		How to get it

----------------------------	-----------------------------------

Starting address			read PMA

Ending address			read PMA or ATIP( to end)

NRA for this track		scan for unrecorded

Write method unknown		TAO PMA

Packet	Fixed + size                      TDB

variable

Audio Data Format			(this & previous track/LI)

unknown

Audio

mode 1

mode 2

mode 2 form 1/2

Track is Complete			Last block written

Copy bit

------------------------------------------------------------------------------

Three commands were identified from the above information.

GET DISC INFO

GET TRACK INFO

READ SPECIAL     -   TOC

-    PMA

-   ATIP

-   SESSION INFO

Bit layout for these will be in future revs of MMC doc.

Other flows modified  were =93Blank Disc=94 now called =91 Empty Disc=94,= Type of Blankness is now called ' Session Status',

6.2   Command Layout

6.2.1.  Review Op-code assignments      -   deferred to next meeting

6.3.  SMC Commands. It was requested that the MMC WG co-ordinate commands, that overlap between SMC and MMC, or define the differences.    Editor to confer with editors of SMC to co-ordinate commands. Report due next meeting.

7.0	New Business

7.1.   Review any Technology proposals (please provide copies of docs, if any)

7.2.   CD-E Technology. Editor requested a proposal to add this technology to the document

8.0  Review of Action Items

Action Item 1)  Rob R. Take ATAPI error codes and incorporate CD-R error codes(make them up)

9.0   Future Meeting Schedule

Next meeting will be held in Manchester NH    Wednesday September 13, 1995  1-6 PM (see meeting map  at hotel) October meeting will be held in Irvine, CA hosted by Western Digital.   Time and location to be published at September meeting.

10.0	 Adjournment

The two day session was adjourned on Thursday 8-3 at 3:30PM.

Attendence List

Name			Company			E-mail

Mr. Chris Brown 		APPLE COMPUTER, Inc		brown.c@applelink.apple.com

Mr. Florey Lin		Cirrus Logic			florey@cirrus.com

Mr. Rick Bohn		Eastman Kodak			ohn@sector.kodak.com

Mr. Charles Yang	Panasonic			yang@got.net

Mr. Kent Manabe		Panasonic/AKEI

Mr. Ted Fujioka		Panasonic/MKE			00207.2272@compuserve.com

Mr. Dave Pohm		Philips				ave-p@lms.com

Mr. Kevin Ross		Philips Semiconductor		ossk@scs.philips.com

Mr. Hiroshi Horii		Sanyo North America		9horii@hr155.a1.sanyo.co.jp

Mr. Masahiko Watabe 	Sanyo Semiconductor Corp

Mr. Ron Roberts		SIERRA-PAC Technology	rkroberts@aol.com

Mr. Mike Yokoyama	SONY				masayuki@cppc.sel.sony.com

Mr. Carl Bonke		Western Digital			bonke@dt.wdc.com

Mr. Devon Worrell	Western Ditigal			worrell@dt.wdc.com

Mr. Tatsuo Fushiki	Yamaha Corp			fushiki@lab1.yamaha.co.jp

Mr. Kasushi Tamai	Yamaha Corp			tama@lab1.yamaha.co.jp

Mr. Masa Morizumi	Yamaha Corp			mori@lab1.yamaha.co.jp

Ron Roberts    SIERRA-PAC TECHNOLOGY

PO BOX 2389     Shingle Springs,   CA

Ph (916) 677-5714       Fax (916) 677-1218

email     rkroberts@aol.com