1

TERMINATOR RESISTOR MISMATCH PROBLEM:			Richard Uber

6-NOV-95

Quantum Corp.

NOMINAL COMPONENT VALUES:

60                         60

+-/\/\/\/-------------------/\/\/\/-+

|                                   |

O                                   O

150   |                                   |  150

1.25v -/\/\/-+                                   +-/\/\/--1.25v

|                                   |

O                                   O

|                                   |

+-/\/\/\/-------------------/\/\/\/-+

60                         60

VOLTAGE SKEW CAUSED BY COMMON MODE OFFSETS AND RESISTOR TOLERANCES

- minimum value for "150 ohm" common mode resistors

- mismatched tolerences on "60 ohm" resistors

- +/- 0.5v ground offsets on terminators

55 min                    65 max

+-/\/\/\/--------+----------/\/\/\/-+

|                |                  |

min   O                |                  O  min

120   |                 +                 |  120

1.75v -/\/\/-+             Voffset               +-/\/\/--0.75v

max          |                 -                 |         min

O                |                  O

|                |                  |

+-/\/\/\/--------+----------/\/\/\/-+

65 max                     55 min

Icommon mode = 1.0v / (120 + 120//120 + 120 ohms)

= 1.0v / 300 ohms = 0.333mA

Voffset = (0.333ma/2 * 65 ohms) - (0.333ma/2 * 55 ohms)

=  108.33mv - 91.66mv

=  16.67mv

This offset can be either polarity.

CONCLUSIONS:

1 A differential voltage offset of +/- 16.67mv is large relative

to system noise margin of only 30mv.

2 On an individual terminator, the "60 ohm" resistors should match

to within +/-  1-2%  instead of the currently specified +/- 8.33%.

TERMINATOR BIAS GENERATOR BEHAVIOR			2

Nominal circuit for idle bus (all devices tristated) = NEGATION

60                         60

+-/\/\/\/---------+---------/\/\/\/-+

|+                |                 |+

O 50-65mv         |                 O 50-65mv

|-                |+                |-

150      |                                   |       150

1.25v -/\/\/-----+         Voffset = 100-130mv       +------/\/\/--1.25v

|+                |-                |+

O 50-65mv         |                 O 50-65mv

|-                |                 |-

+-/\/\/\/---------+---------/\/\/\/-+

60                         60

*******************************************************************************

|\

DRIVEN NEGATED           | >----+ | +5ma

|/     | |

| V

60            |             60

+-/\/\/\/---------+---------/\/\/\/-+

|                 |                 |

|+                |                 |+

O 50-65mv         |                 O  50-65mv

|-                |+                |-

150      |                                   |       150

1.25v -/\/\/-----+       430mv > Voffset > 400mv     +------/\/\/--1.25v

|                                   |

|+                |-                |+

O 50-65mv         |                 O  50-65mv

|-                |                 |-

+-/\/\/\/---------+---------/\/\/\/-+

60            |            60

| |

|\     | |

| >----+ V +5ma

|/

*******************************************************************************

|\

DRIVEN ASSERTED          | >----+ ^ -5ma

|/     | |

| |

60            |             60

+-/\/\/\/---------+---------/\/\/\/-+

|+                |                 |+

O  0-5mv          |                 O   0-5mv

|-                |+                |-

150      |                                   |       150

1.25v -/\/\/-----+      -290mv > Voffset > -300mv    +------/\/\/--1.25v

|+                |-                |+

O  0-5mv          |                 O   0-5mv

|-                |                 |-

+-/\/\/\/---------+---------/\/\/\/-+

60            |            60

| ^

|\     | |

| >----+ | +5ma

|/

3

TERMINATOR MODEL PROBLEM

If the bias generator is modeled as an ideal zener diode as shown

in the draft specifications, then the following situation develops

if there is a common mode voltage difference between the terminators

due to ground offset.

I=278uA

60         ------->       60

+-/\/\/\/---------+---------/\/\/\/-+

|                 |                 |

|+                |                 |+

O  0-5mv          |                 O 50-65mv

|-                |+                |-

150      |                                   |       150

1.40v -/\/\/-----+         Voffset = 50 - 70mv       +------/\/\/--1.20v

|                                   |

--->      |+                |-                |+     ----> I=564uA

I=564uA    O 50-65mv         |                 O  0-5mv

|-                |                 |-

|                 |                 |

+-/\/\/\/---------+---------/\/\/\/-+

60         ----->          60

I=278uA

Because current flows between terminators and not from driver pin

to driver pin, the ideal bias generators split their directions.

Half of them see "negation current" and half of them see "asertion

current".

THE RESULT IS THAT THE PASSIVELY NEGATED BUS SEES ONLY 50-70MV OF

BIAS INSTEAD OF THE 100MV REQUIRED TO INSURE PROPER OPERATION OF

THE LVDS RECEIVERS.

A BETTER TERMINATOR MODEL IS REQUIRED TO ENABLE LVDS PROTOCOL CHIP

DESIGNERS TO DO BUS SIMULATIONS.

4

ALTERNATE TERMINATOR MODEL SUITABLE FOR HSPICE

|

|  |

|  | 10ma

|  v ideal current source

300    |

+--/\/\/---+----/\/\/----------------> Vnn

|          |      60

|          |

|          / 7

|    Rvar  \/

1.25V ---+          / VCR

|         /\ Voltage Controlled

|        / /       Resistor

|          |

|          |      60

+--/\/\/---+----/\/\/----------------> Vpp

300    |

|  |

|  | 10mA

|  v ideal current source

|

The HSpice VCR element can be defined as:

Rvar		Rvar

min		max

+10mv < Vnn - Vpp  		10 ohms		13 ohms

Vnn - Vpp  < -10mv	 0.01 ohms	 1 ohm

+10mv > Vnn - Vpp  > -10mv	linearly	linearly

varies from	varies from

.01 to 10 ohms	1 to 13 ohms

ALTHOUGH THIS DIAGRAM USES "IDEAL HSPICE ELEMENTS" AND IS NOT

PHYSICALLY REALIZABLE, IT DOES MIMIC THE TERMINATOR BEHAVIOR

CLOSE ENOUGH TO ALLOW THE USER TO DO BUS SIMULATIONS.

Also note that there is zero time delay for the terminator to switch

offfset voltage. The Unitrode terminator proposal has an undefined

delay time, but proposes capacitive bypass to allow waves to pass

without delay.

1

FAST-40 NOISE MARGIN

6-NOV-95

Richard Uber

Quantum Corp.

ASSUMPTIONS:

- Receiver sensitivity = +/- 70mv

- Crosstalk for LVDS only bus = 35mv

- Terminator offset = 100 - 130mv favoring negation

= 0 - 10mv when fighting an assertion

- Drive current is 4.4 - 6.0mA

- Terminator resistance = 110 - 130 ohms

- Line impedence = 120 - 160 ohms unloaded

- Loaded line impedence = TBD

Static cases:

state1	state2	Rterm	Vterm	Zline	Idrvr	xtalk	V1	V2	Margin

Negate	  -	110	100mv	 -	4.4ma	 -	 342mv	 -	272mv

Assert	  -	110	 10mv	 -	4.4ma	 -	-232mv	 -	162mv

Z=Negate  -	 -	100mv	 -	 -	 -	 100mv	 -	 30mv

2

FAST-40 NOISE MARGIN CASES

Incident wave transitions

DRIVEN NEGATION ---> HIGH IMPEDENCE NEGATION:

V1=Quiescent Negated voltage--------------------

^	  ^

|	  |  Vswing = Zline * Imax/2

driven voltage		|	  |

Vdrv = I * Rterm/2	|	__v_______________

|	  ^

----------------  100mv	|	  |  noise margin

|	  V

|	----------------------	 Vdiff = 70mv

|	crosstalk allowance = 0 mv

|

|	----------------

terminator		|

offset= 100-130mv	|	rvcr sensitivity = 70mv

v

Vdiff=0 ---------------------------------------------------------    Vdiff = 0

V1 = (min terminator offset) + Imax * (min terminator resistance)/2

Vswing = Zline * Imax / 2

V2 = V1 - Vswing

Noise margin = V2 - (rcvr sensitivity + crosstalk)  = V2 - 70mv

Bus release transients, no crosstalk

state1	state2	Rterm	Vterm	Zline	Idrvr	  V1	Vswing	  V2	Margin

Negate Z=Negate	110	100mv	160	6.0ma	 430mv	-480mv	 -50mv	-120mv

Negate Z=Negate	110	100mv	150	6.0ma	 430mv	-450mv	 -20mv	 -90mv

Negate Z=Negate	110	100mv	140	6.0ma	 430mv	-420mv	  10mv	 -60mv

Negate Z=Negate	110	100mv	130	6.0ma	 430mv	-390mv	  40mv	 -30mv

Negate Z=Negate	110	100mv	120	6.0ma	 430mv	-360mv	  70mv	   0mv

Negate Z=Negate	110	100mv	110	6.0ma	 430mv	-330mv	 100mv	  30mv

Negate Z=Negate	110	100mv	100	6.0ma	 430mv	-300mv	 130mv	  60mv

Negate Z=Negate	110	100mv	 90	6.0ma	 430mv	-270mv	 160mv	  90mv

CONCLUSIONS:

- Cable impedence needs to be limited to 110 ohms MAX.

- This is NOT compatible with current specification of 120 ohms MIN.

- We need to explore other alternatives.

3

ALTERNATIVES FOR BUS RELEASE TRANSITIONS

1 Ignore the state reversals

- Causes serious problems with edge triggered signals REQ, ACK, RST.

- Following synchronous data transfers, hardware which counts

REQ pulses or ACK pulses may still be enabled.

- Causes changes which would ripple far back into chip logic and

into existing software.

- Some vendors implement "bus monitors" which could detect and act

on short assertions on any signal.

- Not acceptable.

2 Turn off half of the driver at a time.

- Would require creating an intermediate state for a bus round trip

time.

state               pin--	pin++

driven negation		+5mA	 -5ma

intermediate mode	+5ma	 Z

passive negation	 Z	 Z

- Requires additional signal, timer in protocol chip core logic

- Injects common mode signal onto bus.

- Requires at least 2 steps to maintain noise margin.

- Not acceptable.

3 Turn off driver in two steps.

- Requires the ability to attenuate drive current to 40-60% of

normal value.

- Would require creating an intermediate state for a bus round trip

time.

state               pin--		pin++

driven negation		+5mA	 	-5ma

intermediate mode	+2 -> +3ma	-2 -> -3ma

passive negation	 Z		  Z

- Requires additional signal, timer in protocol chip core logic

- Does not inject common mode signal onto the bus.

- Requires at least 2 steps to maintain noise margin.

- Not acceptable.

ALTERNATIVES FOR BUS RELEASE TRANSITIONS		4

4 Switch from driving current to driving voltage

- Lessens concern with mismatches between line impedance and

termination impedance.

- Can be done internal to driver, without adding any capacitance

to the SCSI pin.

- The target resistance value can be in a wide range, about

100-300 ohms.

- One implementation could be:

4.4-6.0ma

source

|           |\

| |           | \

| |           |  \

v +-----------|M |

|           |U |------ (-) bus pin

|   +-------|X /

|   |       | /

|   |       |/|

|   |         |

Data -----|---|---------+

|   |         |

|   |       |\O

|   |       | \

|   |       |  \

+---------+-----------|M |

|	      |       |U |------ (+) bus pin

\	      +-------|X /

R	/	      |       | /

\	      |       |/

/	      |

|	      |

/ <--	      |

/	      |

|             | |

+-------------+ |

ATTENUATE             | v

circuit               |

4.4-6.0ma

sink

5

FAST-40 NOISE MARGIN CASES

Incident wave transitions

ASSERTED --> DRIVEN NEGATED:

Incident wave negated voltage ________________________________

^			^

|	noise margin	|

|			v

|	------------------------- 105mv

|	crosstalk allowance = 35mv

|	------------------------- 70mv

-------------------------	|

Terminator offset = 0	  ^	|	Rcvr sensitivity = 70mv

Vdiff = 0 _______________________|_____|____________________________ Vdiff = 0

|	|

Vdrv = I * Rterm/2	  |	| Vswing = I * Zline

|	|

Quiescent Asserted voltage ______v_____v

V1 = (min terminator offset) + Imin * (max terminator resistance)/2

Vswing = Zline * Imin / 2

V2 = V1 - Vswing

Noise margin = V2 - (rcvr sensitivity + crosstalk) =  V2 - 105mv

state1	state2	Rterm	Vterm	Zline	Idrvr	V1	Vswing	V2	Margin

Assert	Negate	130	  0mv	160	4.4ma 	 -286mv	  704mv	 418mv	 313mv

Assert	Negate	130	  0mv	140	4.4ma 	 -286mv	  616mv	 330mv	 225mv

Assert	Negate	130	  0mv	120nom	4.4ma 	 -286mv	  528mv	 242mv	 137mv

Assert	Negate	130	  0mv	110	4.4ma	 -286mv	  484mv	 198mv	  93mv

Assert	Negate	130	  0mv	100	4.4ma	 -286mv	  440mv	 154mv	  49mv

Assert	Negate	130	  0mv	 90	4.4ma	 -286mv	  396mv	 110mv	   5mv

6

FAST-40 NOISE MARGIN CASES

Incident wave transitions

DRIVEN NEGATED --> ASSERTED:

-------------------------- Quiescent Negated voltage = V1

driven voltage		^

Vdrv = I * Rterm/2	|

|

----------------  130mv	|

terminator		|

offset= 100-130mv	|

Vdiff=0 ------------------------|-------------------------------   Vdiff = 0

|	rcvr sensitivity = 70mv

|

assertion	|	-------------------------     = -70mv

Vswing		|	crosstalk allowance = 35mv

= I * Zline	|	_________________________     = -105mv

|	  ^

|	  | noise margin

Incident asserted wave__v_________v_________________________ = V2

V1 = (min terminator offset) + Imin * (max terminator resistance)/2

Vswing = Zline * Imin / 2

V2 = V1 - Vswing

Noise margin = V2 - (rcvr sensitivity + crosstalk) = | V2 - (-105mv) |

state1	state2	Rterm	Vterm	Zline	Idrvr	  V1	Vswing	 V2	Margin

Negate	Assert	130	130mv	160	4.4ma 	 416mv	-704mv	-288mv	 183mv

Negate	Assert	130	130mv	140	4.4ma 	 416mv	-616mv	-200mv	  95mv

Negate	Assert	130	130mv	120	4.4ma 	 416mv	-528mv	-112mv	   7mv

Negate	Assert	130	130mv	110	4.4ma 	 416mv	-484mv	 -68mv	 -37mv

Negate	Assert	130	130mv	100	4.4ma 	 416mv	-440mv	 -24mv	 -81mv

Negate	Assert	130	130mv	 90	4.4ma 	 416mv	-396mv	  20mv	-125mv

Trying larger minimum drive currents:

Negate	Assert	130	130mv	100	5.0ma 	 425mv	-500mv	 -50mv	 -55mv

Negate	Assert	130	130mv	100	6.0ma 	 490mv	-600mv	-110mv	   5mv

Negate	Assert	130	130mv	100	7.0ma 	 555mv	-700mv	-145mv	  40mv

Trying lower termination resistances (45-55 ohms):

Negate	Assert	110	130mv	100	5.0ma 	 375mv	-500mv	-125mv	  20mv

7

RECOMMENDATIONS

- Add a new state to be transitioned through when disconnecting

from the bus. Connect a resistance between the positive and

negative current sources. This resistance is in parallel with

the transmission line, and will permit the SCSI device to exit

from the LVDS bus without any assertion glitches.

- Consider lowering the termination resistance from 60 +/-5 ohms

to 50 +/- 3 ohms.

- Specify tighter matching for differential mode resistors within

a terminator chip.

- Raise drive current range from 4.4 - 6.0mA to 5.0 - 7.0ma

- Limit loaded line impedance to 100 ohms minimum.