Interoffice Correspondence

________________________________________________________________________

Date: 11/6/95

To: John Lohmeyer

Cc:

From: Gene Milligan   OKM251

Subject: SCSI Version Proposal

In a prior working group meeting I proposed that SCSI include information reporting which version of the family of SCSI standards had been implemented in the product. I described how that might be done using a technique analogous to the one that has been included in ATA-3. The group recommended that I document the proposal for inclusion in SPC.

The attached modification of the INQUIRY command response is a documentation of the proposal. I have a file with the changes in complete context. However for brevity the attached includes only the changed portions.

Gene E. Milligan

7.5 INQUIRY command

7.5.1 Standard INQUIRY data

Table 19 - Standard INQUIRY data format

+=====+========+========+========+========+========+========+========+========+

|  Bit|   7    |   6    |   5    |   4    |   3    |   2    |   1    |   0    |

|Byte |        |        |        |        |        |        |        |        |

+=====+========+========+========+========+========+========+========+========+

| 2   |   ISO version   |       ECMA version       |  ANSI version            |

+-----+--------+--------+--------+--------+--------+--------------------------+

| 56  | (MSB)                                                              |

+- - -+- -                         Interface type                       - -+

| 57  |                                                              (LSB) |

+-----+--------------------------------------------------------------------+

| 58  | (MSB)                                                              |

+- - -+- - 	                      Major version number                   - -+

| 59  |                                                              (LSB) |

+-----+--------------------------------------------------------------------+

| 60  | (MSB)                                                              |

+- - -+- - 	                     Physical version  number                - -+

| 61  |                                                              (LSB) |

+-----+--------------------------------------------------------------------+

| 62  | (MSB)                                                              |

+- - -+- - 	              Primary Command set version  number            - -+

| 63  |                                                              (LSB) |

+-----+--------------------------------------------------------------------+

| 64  | (MSB)                                                              |

+- - -+- -          Device Type Command set version  number             - -+

| 65  |                                                              (LSB) |

+=====+========================================================================+

| 66  |                                                                        |

+- - -+- -                        Reserved                                  - -+

| 95  |                                                                        |

+=====+========================================================================+

The ANSI-approved version field indicates the implemented version of this standard and is defined in table 22

Table 22 - ANSI-approved version

+=============+===============================================================+

|    Code     |  Description                                                  |

+-------------+---------------------------------------------------------------+

|      0h     |  The device might or might not comply to an ANSI              |

|             |  standard.                                                    |

|      1h     |  Reserved for historical uses                                 |

|      2h     |  The device complies to ANSI X3.131-1994 (SCSI-2).            |

|      3h     |  The device complies to this or a later version of SCSI.    |

|             |   This code is reserved to designate this standard upon       |

|             | approval by ANSI.                                             |

|   4h - 7h   |  Reserved                                                     |

|             |                                                               |

+=============+===============================================================+

7.5.4 Command support data

Table 25 - Command support data format

+=====+========+========+========+========+========+========+========+========+

|  Bit|   7    |   6    |   5    |   4    |   3    |   2    |   1    |   0    |

|Byte |        |        |        |        |        |        |        |        |

+=====+========+========+========+========+========+========+========+========+

| 2   |   ISO version   |       ECMA version       |  ANSI version            |

+-----+-----------------+--------------------------+--------------------------+

If the operation code being tested is supported as defined in a SCSI standard, the StdOp bit shall be one, the VSop bit shall be zero, and the ISO, ECMA and ANSI-approved version fields shall contain standard INQUIRY data naming the standard that defines the SCSI command.  (Information about standard INQUIRY data can be found in clause .)  If the operation code being tested is supported in a vendor-specific way, the StdOp bit shall be zero, the VSop bit shall be one, and interpretation of the ISO, ECMA and ANSI-approved version fields by the application client shall be vendor-specific.  If the operation code being tested is not supported, both the StdOP and VSop bits shall be zero.

#### Bytes 56 & 57: Interface type

If not 0000h, the device claims compliance with the interface type and major version of that interface type.

The types specified are shown in Table ??. All other values are reserved.

Table ??- Interface type0

| Value | Interface type |
| --- | --- |
| 0100h | SPI device |
| 0200h | FCP device |
| 0300h | SSA device |
| 0400h | SBP device |

#### Bytes 58 & 59: Major version number

If not 0000h, the device claims compliance with the major version(s) as indicated by bits 1 through 3 being equal to one. Values other than 0000h are bit significant. Since the SCSI-3 and SCSI-2 standards maintain downward compatibility with SCSI-1 (published as SCSI), it is allowed for a  SCSI-3 device to set all of bits 1 through 3 to one.

Table ?? Major version number0

| Bit Number | Value | Interface type |
| --- | --- | --- |
| 0 | 1 | SCSI |
| 1 | 1 | SCSI-1 |
| 2 | 1 | SCSI-2 |
| 3 | 1 | SCSI-3 |
| 4-15 | 0 | Reserved |

#### Bytes 60 & 61: Physical version  number

If an implementor claims that the revision of the standard they used to guide their implementation does not need to be reported or if the implementation was based upon a standard prior to this revision of the standard, Words 60 & 61 shall be 0000h.

A revision of the standard which guided the implementation may optionally be reported in Words 60 & 61 as shown in Table ??.

Table ??- Physical version  number0

| Value | Minor revision (this table may need columns added for some physicals) |
| --- | --- |
| 0001h | SCSI |
| 0003h | SCSI-2 prior to revision 10h |
| 0005h | SCSI-2 revision 10h |
| 0007h | SCSI-2 as published |
| 0009h | SPI-2 prior to revision 15a |
| 0008h | SPI-2 revision 15a |
| 0006h | SPI-2 as published |
| 0002h | SCSI Fast-20 revision 6 |
| 0004h | SCSI Fast-20 as published |
| 000Ah | FC-AL revision 4.5 & FC-PH as published |
| 000Bh | FC-AL as published & FC-PH as published |
| 000Ch | FC-AL revision 4.5 & FC-EP Revision TBD |
| 000Dh | FC-AL as published & FC-EP Revision TBD |
| 000Eh | FC-PH as published |
| 000Fh | FC-EP Revision TBD |
| 0010h | SSA-PH1 revision TBD |
| 0011h | SSA-PH1 as published |
| 0012h | SSA-PH2 revision TBD |
| 0013h | SSA-PH2 as published |
| 0014h | IEEE 1394 revision D7.1 |
| 0015h | IEEE 1394 as published |
| 0016h-FFFFh | Reserved |

#### Bytes 62 & 63: Primary Command set version  number

If an implementor claims that the revision of the standard they used to guide their implementation does not need to be reported or if the implementation was based upon a standard prior to this revision of the standard, Words 62 & 63 shall be 0000h.

A revision of the standard which guided the implementation may optionally be reported in Words 62 & 63 as shown in Table ??.

Table 1?? - Primary Command set version  number1

| Value | Minor revision |
| --- | --- |
| 0001h | SCSI-1 |
| 0003h | SCSI-2 section 7 prior to revision 10h |
| 0005h | SCSI-2 section 7 revision 10h |
| 0007h | SCSI-2 section 8 as published |
| 0009h | SPC Revision 4 |
| 0008h | SPC Revision 5 |
| 0006h | SPC Revision 6 |
| 0002h | SPC Revision 7 |
| 0004h, 000Ah-FFFFh | Reserved |

#### Bytes 64 & 65: Device Type Command set version  number

If an implementor claims that the revision of the standard they used to guide their implementation does not need to be reported or if the implementation was based upon a standard prior to this revision of the standard, Words 64 & 65 shall be 0000h.

A revision of the standard which guided the implementation may optionally be reported in Words 64 & 65 as shown in Table ??.

Table ?? - Device Type Command set version  number1

| Value | Minor revision (this table may need columns added for some physicals) |
| --- | --- |
| 0001h | SCSI-1 |
| 0002h | SCSI-2 section 8 prior to revision 10h |
| 0005h | SCSI-2 section 8 revision 10h |
| 0007h | SCSI-2 section 9 as published |
| 0009h | SCSI-2 section 9 prior to revision 10h |
| 0008h | SCSI-2 section 9 revision 10h |
| 0006h | SCSI-2 section 10 as published |
| 0002h | SCSI-2 section 10 prior to revision 10h |
| 0004h | SCSI-2 section 10 revision 10h |
| 000Ah | SCSI-2 section 11 as published |
| 000Bh | SCSI-2 section 11 prior to revision 10h |
| 000Ch | SCSI-2 section 11 revision 10h |
| 000Dh | SCSI-2 section 12 as published |
| 000Bh | SCSI-2 section 12 prior to revision 10h |
| 000Ch | SCSI-2 section 12 revision 10h |
| 000Dh | SCSI-2 section 13 as published |
| 000Eh | SCSI-2 section 13 prior to revision 10h |
| 000Fh | SCSI-2 section 13 revision 10h |
| 0010h | SCSI-2 section 14 as published |
| 0011h | SCSI-2 section 14 prior to revision 10h |
| 0012h | SCSI-2 section 14 revision 10h |
| 0013h | SCSI-2 section 15 as published |
| 0014h | SCSI-2 section 15 prior to revision 10h |
| 0015h | SCSI-2 section 15 revision 10h |
| 0016h | SCSI-2 section 16 as published |
| 0017h | SCSI-2 section 16 prior to revision 10h |
| 0018h | SCSI-2 section 16 revision 10h |
| 0019h | SCSI-2 section 17 as published |
| 001Ah | SCSI-2 section 17 prior to revision 10h |
| 001Bh | SCSI-2 section 17 revision 10h |
| 001Ch | SCSI-2 section 18 as published |
| 001Dh | SCSI-3 SBC Revision 2 |
| 001Eh | SCSI-3 SBC as published |
| 001Fh | SCSI-3 SCC Revision 5 |
| 0020h | SCSI-3 SCC Revision 6 |
| 0021h | SCSI-3 SCC as published |
| 0022h | SCSI-3 SSC Revision 2 |
| 0023h | SCSI-3 SSC Revision 3 |
| 0024h | SCSI-3 SSC as published |
| 0025h | SCSI-3 SGC Revision 0 |
| 0026h | SCSI-3 SGC as published |
| 0027h | SCSI-3 SMC Revision 3 |
| 0028h | SCSI-3 SMC as published |
| 0029h | SCSI-3 MMC Revision 2b |
| 002Ah | SCSI-3 MMC Revision 3 |
| 0030h | SCSI-3 MMC as published |
| 0031h-FFFFh | Reserved |