X3, Information Processing Systems

Doc:    X3T10/96-155R0

Date:   April 11, 1996

Project:X3T10/997-D

Ref. Doc.: 997R06

Reply to: Ken Hallam

Unisys

ken.hallam@mv.unisys.com

To:      X3T10 SSC Working Group Membership

Subject: Proposal for SSC Log Page 0Ch

Problem:

The SSC draft provides a way for tape drives to inform the host that cleaning is required by means of Log Page code 0Ch, (Sequential-access Device Log Page) and Log Parameter code of  0100h. A non zero value in the Log Parameter returned under this code indicates that the drive requires cleaning. However, there is no guidance in the draft regarding the length of the parameter. Some existing models of tape products in the field today use parameter code 0100h with a parameter length of 8 bytes, while others use a parameter length of 2 bytes. A variable length for this parameter value is a problem. Parsing a multiple byte field to check for non-zero content is not efficient. In addition, it is difficult to pre-allocate space for a variable field in software that must deal with many different models of tape drives.

Proposal:

Modify the paragraph following Table 26 in clause 5.3.2.1 of draft 997 by adding the following text to the end of the paragraph:

“The parameter value returned under parameter code 0100h shall be a 2 byte field, thus byte 3 of the Log Parameter header shall be set to 02h.”

Table 26

| Parameter Code | Description |
| --- | --- |
| 0000h | Number of data bytes received from application |
|  | clients during WRITE command operations. |
| 0001h | Number of data bytes written to the media as a |
|  | result of WRITE command operations, not counting ECC |
|  | and formatting overhead. |
| 0002h | Number of data bytes read from the media during READ |
|  | command operations, not counting ECC and formatting |
|  | overhead. |
| 0003h | Number of data bytes transferred to Data-Out buffers |
|  | during READ command operations. |
| 0004h - 00ffh | Reserved |
| 0100h | Cleaning required |
| 0101h - 7FFFh | Reserved |
| 8000h - FFFFh | Vendor-specific parameters |

A non-zero value of the cleaning required parameter indicates that a condition requiring cleaning has been detected and a subsequent cleaning cycle has not been completed. The cleaning required parameter shall be persistent across hard resets and power cycles. “The parameter value returned under parameter code 0100h shall be a 2 byte field, thus byte 3 of the Log Parameter header shall be set to 02h.”

Regards,

Ken Hallam

Unisys

ken.hallam@mv.unisys.com