Peter Johansson

Congruent Software, Inc.

07MAY96

SAM
and
Simple Task Management

SAM today has a simple dichotomy: either the target supports tagged tasks or it doesn’t.

A third model, of support for tagged tasks with the SIMPLE task attributes, is proposed because it adds value to the desktop environment .

What’s needed to support simple tagged tasks (a.k.a. simple task management)?

Simple Task Management

A SCSI-3 target that supports simple task management shall:

- implement the behaviors described by SCSI-3 Primary Commands (SPC) for a control mode page (0A16) value of one for the queue algorithm modifier field (unrestricted reordering),

- implement the behaviors described by SPC when the QErr bit in the control mode page is set to one (task set cleared when ACA is cleared),

- accept one untagged task at a time (the task is treated as if it has a task attribute of SIMPLE),

- accept tagged tasks with a task attribute of SIMPLE,

- reject tasks with a task attribute of ORDERED or HEAD OF QUEUE,

- accept the CLEAR TASK SET and TARGET RESET task management functions,

- reject the ABORT TASK, ABORT TASK SET and TERMINATE TASK task management functions, and

- support the CLEAR ACA task management functions.

SAM Changes

Define simple task management model:

- the only task management functions supported are CLEAR ACA, CLEAR TASK SET and TARGET RESET,

- the only task attribute supported is SIMPLE,

- the target behavior is constrained so QErr equals one and unrestricted reordering of the SIMPLE tasks is not just permitted but required