Accredited Standards Committee*

X3, Information Processing Systems

Doc. No.    X3T10/96-193R0
Date:	     November 8, 1995
Project:     X3T10-1048D	
Ref. Doc.:  X3T10/94-057

Reply to:   R. Roberts

To:		Membership of X3T10

From:	R. Reisch/R. Roberts

Subject:	Minutes of X3T10 MMC  Working Group - July 16,17, 1996

Agenda

1.0      Opening Remarks

2.0      Introductions

3.0      Document Distribution

4.0      Call for Patents

5.0      Approval of Agenda

6.0      Meeting Agenda

6.1 - Review Mike Yokoyama (SONY) comments to Rev 7.0G.

6.2 - Review John Hanmann (Western Digital ) comments.

6.3 - Reviewed Tony Pione (Oak Technology) comments.

6.4 - Reviewed Masa Morizumi (Yamaha) comments.

6.5 - Reviewed Rob Sims (HP) comments.

6.6 - Reviewed Ricoh's comments.

6.7 - Yamaha presented two pages of additional comments.

6.8 - Mike Yokoyama presented Ricoh's comments Blank command.

6.9 - Reviewed change bars in rev 7.0H

7.0      New Business

8.0      Review of Action Items

9.0      Future Meeting Schedule

10.0     Adjournment

Minutes

1.0     Opening Remarks

R. Roberts called the meeting to order at 9:00 AM on Tuesday 7/16/96. He thanked Symbios for hosting the meetings. Ron asked Mr. Rob Reisch of Kodak to act as recording secretary for the meetings.

2.0     Introductions/Attendance

Introductions of attendees were made and a sign-up sheet was distributed. List of attendees is attached.

3.0     Document Distribution

Philips distributed Revision  7.0G both hard copies and electronic copies of the document were distributed to attendees. Copies of comments from Mike Yokoyama (SONY) distributed copies of comments from SONY and RICOH.

Ron Roberts distributed a copy of the new project proposal for MMC-2.

Ron explained his plan to incorporate all new comments into rev 8.0 that will be presented at the Plenary Meeting Thursday 7/18/96.

4.0     Call for Patents

The normal request for patent disclosures was made. No patents were identified.

5.0     Approval of Agenda

No modifications were made.

6.0     Meeting Agenda

6.1     Review Mike Yokoyama (SONY) comments to Rev 7.0G.

6.1.1   First byte in Table 63 was added. The first byte is the same as byte 4 of table 62.

6.1.2   Agreed to remove default Write type as track at Once. This was the only default 
		parameter specified.

6.1.3   Open issue, E-mail to Ricoh.

6.1.4   Modified read buffer capacity by deleting "only" and adding TAO is an option. The 
		modifications were accepted.

6.1.5   Accepted the use of the error code defined 02/04/04  as  "in the process of becoming 
		ready, writing." for all commands.

6.1.6   Same as 5.0 accepted.

6.1.7   Same as 5.0 accepted.

6.1.8   Accepted with the following modifications. "If this is a partially recorded recorded reserved 
		track or an empty reserved track, the drive shall pad the track."

6.1.9   Accepted with no modifications.

6.1.10  Accepted with minor editorial changes.

6.1.11  Modified the proposal to clarify that the FP bit is for the result data from the Read Track 
		Info command. Also, the free blocks field shall report a value assuming variable packets. 
		Accepted with following modifications: "When RT, Blank and Packet flags are set to one, 
		FP flag of a real Track Information result data is set to zero." After table 139 add
		 following sentence: "If this track is the first track of a session the session at once 
		recording is allowed."

6.1.12  Add to table 139 the following bit definitions:

RT. Blank. Packet.

0.       0.       0        Can't write to stamped disc or during track at once on invisible track 
				      or writing session at once mode.

1.       0.       0        Can't write to recorded track or during Track at Once on reserved 
				      track.

6.1.13  Modified the change to table 140 as follows:    Value       Definition   " Fh        Data block 
		type is unknown(no Track Descriptor Block)"

6.1.14  Add table ??? Track Status Indications, with the following content:
		RT     Blank   Packet  FP            Write Method                   Track Status

0         0         0          -             Uninterrupted TAO SAO   Complete during TAO/SAO

0         0         1          0             Variable                            Incomplete

0         0         1          1             Fixed                                Incomplete

0         1         0           -             TAO/Variable/Fixed(*)      Invisible

0         1         1           0             -                                       Invalid

0         1         1           1             -                                       Invalid

1         0         0            -             TAO                                Complete/During TAO

1         0         1           0             Variable            Complete/ Partially recorded Reserve

1         0         1           1              Fixed               Complete/ Partially recorded Reserve

1         1         1           -               TAO                                Empty Reserved

1         1         1           0             Variable/Fixed                  Empty Reserved

1         1         1           1              -                                      Invalid

* In case last session is empty, SAO is also valid.

6.1.15  Page 121 Table 141 Next Writable Address Definition Delete "This address is dependent 
		upon the setting of Write.  in Table 141."  
		Add    "Next Writable Address is independent upon the setting of Write Type in Write 
		Parameters Mode page.  It shall be associated with the RT, Blank, Packet and FP flags 
		as defined in Table XXX".

Table XXX: Next Writable Address

==============================================================================

RT + Blank + Packet + FP + NWA_V    +          definition of Next Writable Address

------------------+-----------------------------------------------------------

0        0            0           -       0(*4)               LBA that shall be specified by next write command(*2)

0        0            1           0       1(*1)              LBA that shall be specified by next write command(*2)

0        0            1           1       1(*1)              LBA that shall be specified by next write command(*2)(*3)

0        1            0            -       1                   LBA of the first data block after pre-gap

0        1            1           0        -                   -

0        1            1           1        -                   -

1        0            0            -       0(*4)             LBA that shall be specified by next write command(*2)

1        0            1           0       1(*1)             LBA that shall be specified by next write command(*2)

1        0            1           1       1(*1)             LBA that shall be specified by next write command(*2)(*3)

1        1            0            -       1                  LBA of the first data block after pre-gap

1        1            1           0       1                  LBA of the first data block after pre-gap

1        1            1           1        -                   -

=============================================================================

*1: When "Free Blocks" is 0 (data full), NWA_V is 0.

*2: NWA shall be taken account of data blocks in buffer that has not yet been written to media.  If the 
	drive can write the data of next write command without interrupting of current data streaming
 	(no underrun condition), NWA shall be contiguous to last address data in buffer. If WCE in Mode 
	Cache Page is zero, NWA shall be taken account of Link Blocks (2 Run-out blocks, 1 Link block 
	and 4 Run-out blocks) in case of Addressing Method-1.

*3: NWA shall follow the Addressing Method-2 if Method-2 flag in Mode CD Capabilities and 
	Mechanical Status Page is set to one.

*4: During TAO (SAO), NWA_V is 1.

6.1.17  Page 123 the 2nd paragraph from the bottom. The paragraph will be changed:
	 "Reserving shall be allowed when the track is invisible.  Attempting to reserve an existing 
	incomplete track shall cause a CHECK CONDITION status, ILLEGAL REQUEST, COMMAND 
	SEQUENCE ERROR."

6.1.18  Page 123 "If the reserved size is smaller than 4 seconds excluding pregap length , drive 
	shall return a CHECK CONDITION STATUS and the sense is set to a sense key of 05, ASC of 
	24 and a ASCQ of 00, Invalid field in CDB.

6.1.19  Page 136, six paragraph second sentence to the following:  "In case of  WCE=1(Write 
	Cache Page) and FUA=0 with variable packet writing, if the LBA is equal to the Next Writable 
	Address in the same track as a previous Write, writing should continue without interruption of 
	streaming."

6.1.20  Page 137, make the following changes to item 4 READ TRACK INFO:  "4)  READ 
	TRACK INFO (for current track). If the LBA specified is not within the current track, the drive 
	may return a CHECK CONDITION Status with the sense data set to a key of 05, ASC of 24 and 
	ASCQ of 00, Invalid field in CDB”

6.1.21  Page 155 change Invisible Area  to Invisible Track. Search document for any references 
	to Invisible Area and change to Invisible Track.

6.1.22  Page 160 the following changes were made to Figure 0.5:
		-Two places EFW changed to EFM
		-Bottom of figure change NO to YES and YES to No.
		-Add the following note below figure 0.5:  "Note: Sectors that contain sub Q mode 0 shall 
		be considered as no EFM."

6.2     Review John Hanmann (Western Digital ) comments.

6.2.1.  Page 144, All control bytes are reserved and set to zero.

6.2.2   Table B.1 ATAPI CD Commands make the following changes: 
		-Mechanism Status changed to mandatory.
		-Remove Seek and Test Unit Ready.	
		-Stop Play/Scan will be mandatory.

6.2.3   Modify Table 11 as follows (changes are made so that ATAPI and MMC are consistent):
		-Delete seek. 
		-Mechanism Status will be Mandatory 
		-Set CD Speed will be R and add definition for R,  Mandatory for CDR/E. 
		-Stop Play/Scan will be A.

6.2.4   Page 78 between Table 86 and 87 add the following new paragraph: "Issuing a 
		stop play/scan command when no play operation is in progress shall not be considered 
		an error."

6.2.5   Page 77 Set CD Speed command make the following changes:
		-Table 84 bytes 2 &3 changed Drive speed to Drive read speed.
		-Table  84 byte 5 add "(reserved for CD ROM readers)."
		-Between table 84 and 85 make the following changes: "The Drive Write and Read 
		Speed parameters contain the requested data rates the drive should use.  The device 
		may choose to select the speed specified or any slower rate.  A value of FFFFh will set 
		the Drive Speed to the maximum supported.  Requesting a speed faster than the drive 
		supports shall not generate an error.  The actual speed set is returned in the Capabilities 
		Mode Sense page.(See Table 100- CD Capabilities and Mechanical Status Page)."

6.2.6   Page 103, Table 112 the following changes were made:
		-Code 001, the description, last sentence, "used" was replaced with "exercised". 
		-Code 010, the description, last sentence was replaced with the following:
		 "The track to be erased shall be in an incomplete session."
		-Code 011b, the description, first sentence is changed as follows: "This is valid only 
		when the last recorded track is incomplete, reserved, or is complete and in an 
		incomplete session."
		-Code 100b, description, third and forth sentences are changed as follows: "This blank 
		type is valid for only a Packet track. This may be used to prepare for writing to a packet 
		track to a CD-E disc with the same write process as a CD-R.  The track to be erased
		shall be in an incomplete session."

6.2.7   Page 108, last sentence first paragraph is to be modified and made into a separate 
		paragraph as follows:  " If the last session on a disc is not complete when this command 
		is issued, a CHECK CONDITION status with the sense data set to a key of  05, ASC of 
		2C and ASCQ of 00, Command Sequence Error."

6.2.8   Page 108 last paragraph, is changed as follows: "The session bit shall be ignored when 
		the Grow bit is set."

6.2.9   Page 142, Add the following error code to Table A.1.:

Sense Key     ASC    ASCQ                   Description
		    05               10         05                 Current Program Area is not empty. The program 
						       area of the current session is written and does 
						       contain reserved tracks.

Page 143 Add the following error code to Table A.1:

Sense Key     ASC    ASCQ                   Description

05                71         05                 Can not write session at once, no cue sheet.

6.3     Reviewed Tony Pione (Oak Technology) comments.

6.3.1   Pages 145, to table B.1 make the following changes: Format Unit, Operation code 04h., 
		type O.

6.3.2   Page 101 Table 110 the following changes were made: Format Unit command 
		Operational Code 04h.

6.4     Reviewed Masa Morizumi (Yamaha) comments.

6.4.1   Page  97, Table 105 Packet/Session Size changed to Packet Size.

6.4.2   Page 104 changes made to specification.

6.4.3   Page 125 changes made to specification.

6.4.4   Page 126 changes made to specification.

6.4.5   Page 148 changes made to specification.

(details for above in editors marked up spec.)

6.5     Reviewed Rob Sims (HP) comments.

6.5.1   Page 134 send OPC, a bit was added to CDB to support Do OPC.  See rev 8 for details.  
		Rob had further comments that  occurred during all of the reviews of other comments.

6.6     Reviewed Ricoh's comments

6.6.1   Already taken care of.

6.6.2   Format Unit Command changes were made, Neil please review the changes 
	submitted.

6.6.3   CLV recording speed changed to 4 bits.

6.6.4   Blank definition has been further defined such that a track that has just the pre-
	gap recorded shall not be considered to be blank.

6.6.5   The group would not support a open, re-open function within Close Track/Session.

6.6.6   Blank command, Erase Session (blanking type=110b) will be an  optional part of 
	the command.

6.7     Yamaha presented two pages of additional comments

6.7.1   Yamaha presented New formulas  for page 121 on the free blocks for a fixed packet track 
	and free blocks for the invisible track.

6.7.2   Page 104, after Table 115 remove the next paragraph.

6.7.3   Page 119 Damage  flag, add the following: "When the damage  ...... the drive may ......"

6.7.4   Small editorial change to sample que sheet.

6.8      Mike Yokoyama presented Ricoh's comments concerning Blank command and Format Unit 
	command. After a brief discussion the specification was modified.

6.9     Reviewed change bars in Rev 7.0H that will become Rev 8.0.

6.9.1   Add 02/04/04 to all command error tables. (added to Rev 8.0)

6.9.2   Table 63 64 and 65 needs further modification.(added to Rev 8.0)

6.9.3   Page 103 add Ricoh's Blank session function.(110b).(added to Rev 8.0)

6.9.4   Page 121, new formulas for caculating free blocks.(added to Rev 8.0)

6.9.5   Recemonded sense data table missing from the following commands:

-Send OPC (added to Rev 8.0)

-Synchronize Cache  (added to rev 8.0)

7.0     New Business

7.1     A vote was conducted to recommend the forwarding Rev 8.0, with change bars from the July 
	working group meeting, for further processing by X3T10. The vote was unanimous to forward the 
	document ! ! ! ! !(11-0)

8.0     Review of Action Items

-The group to actively review Rev 8.0.

-Neil S. and Bill M. review the Blank Session functionality added to Rev 8.0.

9.0     Future Meeting Schedule

- No MMC August working group meeting.

- MMC September working group meeting scheduled for September 10 and 11 1996, Crowne 
	- Plaza, Natick MA.

10.0    Adjournment

The meeting adjourned at 4:30 PM on July 17th.

***************************************************************************************************

Rob Reisch            Eastman Kodak

460 Buffalo Rd. Rochester N.Y.14652-3816

Phone:716-588-0573 
E-mail: reisch@kodak.com

************************************************************************************************

Ron Roberts         Sierra-PAC Technology

PO Box 2389      Shingle Springs, CA 95682

PH 916-677-5714     Fax  916-677-1218

E-mail:  rkroberts@ao;.com

***************************************************************************************************

July 16, 17, 1996    Attendance List

| Name | Company | E-mail |
| --- | --- | --- |
| Mr. Kent Manabe | Amer Kotabuki Elect Ind. | 102511.1602@compuserve.com |
| Mr. Ken Wolfgang | Cirrus Logic | kenw@colorado.cirrus.com |
| Mr. Robert Reisch | Eastman Kodak | reisch@sector.kodak.com |
| Mr. Rob Sims | Hewlett Packard | robsims@depeche.lvld.hp.com |
| Mr. Jeff Kishida | JVC Information Products | 75022.3003@ compuserve.com |
| Mr. Anthony E. Pione | Oak Technology | AnthonyP@oaktech.com |
| Mr. Bill McFerrin | Philips | bmcferrin@aol.com |
| Mr. Ron Roberts | SIERRA-PAC Technology | rkroberts@aol.com |
| Mr. Mike Yokoyama | SONY | masayuki@cppc.sel.sony.com |
| Mr. Tatsuya Imokuchi | SONY | tatsuya@strg.sony.co.jp |
| Mr. Shigeki Tsukatani | SONY | tukatani@strg.sony.co.jp |
| Mr. Jonathan Hanmann | Western Digital | hanmann.j@a1.wdc.com |
| Mr. Masa Morizumi | Yamaha Corp | mori@lab1.yamaha.co.jp |

*Operating under the procedures of The American National Standards Institute.

X3 Secretariat, Computer and Business Equipment Manufacturers Association (CBEMA)

1250 Eye Street NW, Suite 200, Washington, DC 20005-3922

Telephone: 202-737-8888 (Press 1 twice) FAX: 202-638-4922 or 202-628-2829