CONGRUENT SOFTWARE, INC.
3998 Whittle Avenue
Oakland, CA  94602
(510) 531-5472
(510) 531-2942 FAX

FROM:	Peter Johansson

TO:	X3T10 SCSI-3 Working Group

DATE:	September 9, 1996

RE:	Enhanced Configuration ROM for SBP-2 Devices

If SBP-2 is to be able to support devices that implement a diversity of command sets, provision must be made within configuration ROM to describe the software interfaces to these command sets.

The attached document is a proposed replacement for clause 7.3, Unit directory, in Revision 1b of the SBP-2 working draft. The proposal adds new entry types to the unit directory that permit the specification of an organization responsible for both the definition of the command set and the command set version used by the unit. In addition a new entry is defined to permit the specification of logical unit characteristics.

The pages that follow reproduce the current clause 7.3 and show change bars for the proposed modifications.

### Unit directory

Configuration ROM for SBP-2 units shall contain a unit directory in the format specified by this standard. The unit directory shall contain Unit_Spec_ID and Unit_SW_Version entries, as specified by ISO/IEC 13213:1994.

The SBP-2 unit directory shall additionally contain a Command_Set_Spec_ID, Command_Set_Version, Management_Agent and Unit_Characteristics entry and one or more Logical_Unit entries, as defined by this standard.

#### Unit_Spec_ID entry

The Unit_Spec_ID entry is an immediate entry in the SBP-2 unit directory that specifies the organization responsible for the architectural definition of the unit. Figure 45 shows the format of this entry.

Figure 45 – Unit_Spec_ID entry format

00 609E16

most significant

least significant

1216

1216 is the concatenation of key_type and key_value for the Unit_Spec_ID entry.

00 609E16 is the unit_spec_ID obtained from the IEEE/RAC. The value indicates that the X3 Secretariat is responsible for the software interface definition.

#### Unit_SW_Version entry

The Unit_SW_Version entry is an immediate entry in the SBP-2 unit directory that, in combination with the unit_sw_version, specifies the software interface of the unit. Figure 46 shows the format of this entry.

Figure 46 – Unit_SW_Version entry format

01 048316

most significant

least significant

1316

1316 is the concatenation of key_type and key_value for the Unit_SW_Version entry.

01 048316 is the unit_sw_version value that indicates that the unit conforms to this standard.

#### Command_Set_Spec_ID entry

The Command_Set_Spec_ID entry is an immediate entry in the SBP-2 unit directory that specifies the organization responsible for the command set definition for the unit. Figure 47 shows the format of this entry.

Figure 47 – Command_Set_Spec_ID entry format

command_set_spec_ID

most significant

least significant

3816

3816 is the concatenation of key_type and key_value for the Command_Set_Spec_ID entry.

The command_set_spec_ID is an organizationally unique identifier obtained from the IEEE/RAC. The organization to which this 24-bit identifier has been granted is responsible for the definition of the command set implemented by the unit.

#### Command_Set_Version entry

The Command_Set_Version entry is an immediate entry in the SBP-2 unit directory that, in combination with the command_set_spec_ID, specifies the command set implemented by the unit. Figure 48 shows the format of this entry.

Figure 48 – Command_Set_Version entry format

command_set_version

most significant

least significant

3916

3916 is the concatenation of key_type and key_value for the Command_Set_Version entry.

The meaning of command_set_version shall be specified by the owner of command_set_spec_ID.

#### Management_Agent entry

The Management_Agent entry is an immediate entry in the SBP-2 unit directory that specifies the base address of the Unit’s management agent CSR. Figure 49 shows the format of this entry.

Figure 49 – Management_Agent entry format

csr_offset

most significant

least significant

7A16

7A16 is the concatenation of key_type and key_value for the Management_Agent entry.

The csr_offset field shall contain the quadlet offset, from the base address of initial register space, FFFF F000 000016, to the base address of the MANAGEMENT_AGENT register for the unit. All SBP-2 unit CSR’s shall be located at or above address FFFF F001 000016; therefore the value of csr_offset shall not be less than 400016.

#### Unit_Characteristics entry

The Unit_Characteristics entry is an immediate entry in the SBP-2 unit directory that specifies characteristics of the unit implementation. Figure 50 shows the format of this entry.

Figure 50 – Unit_Characteristics entry format

q

most significant

least significant

3916

o

i

reserved

ORB_size

3B16 is the concatenation of key_type and key_value for the Unit_Characteristics entry.

The q bit shall specify the task management (queuing) model implemented by the unit. If q is clear, the unit implements the basic task management model defined by this standard in 10.2. Otherwise the unit implements the full task management model.

The ordered bit (abbreviated as o in the figure above) specifies the implicit task attribute implemented by the SBP-2 unit. If the unit implements SIMPLE tasks, only, the ordered bit shall be clear. Otherwise, if the unit implements ORDERED tasks, only, the ordered bit shall be set. When ordered is set, the unit shall both execute tasks in order and report their completion status in the same order.

The isochronous bit (abbreviated as i in the figure above) specifies whether or not the SBP-2 unit supports isochronous operations. When isochronous is set, isochronous login requests, stream CDB requests and stream control requests shall all be supported. If the isochronous bit is set, the irmc, cmc and isc bits in the bus information block shall also be set, as described in 7.1.

The ORB_size field shall specify, in quadlets, the fetch size used by the SBP-2 unit to obtain ORB’s from initiator memory. The initiator shall allocate, on a quadlet aligned boundary, at least this much memory for each ORB signaled to the target.

-

-

Logical_Unit entry

The Logical_Unit entry is an immediate entry in the SBP-2 unit directory that specifies characteristics of logical units implemented by the SBP-2 device. There shall be at least one Logical_Unit entry in the unit directory for logical unit zero. Additional Logical_Unit entries are optional and may be used by SBP-2 devices that implement multiple logical units. Figure 51 shows the format of this entry.

Figure 51 – Logical_Unit entry format

most significant

least significant

3C16

lun

r

device_type

e

3C16 is the concatenation of key_type and key_value for the Logical_Unit entry.

The ejectable bit (abbreviated as e in the figure above) shall specify whether or not the unit medium is removable. When ejectable is set it indicates that the medium is removable.

The device_type field indicates the peripheral device type implemented by the logical unit. This field shall contain a value as specified by SPC for Peripheral Device Type.

The lun field shall identify the logical unit to which the information in the Logical_Unit entry applies.