Accredited Standards Committee*

X3, Information Technology

Doc. No.:	X3T10/96-227r0

Date:	August 14, 1996

Project:	1142-D

Ref. Doc.:

Reply to:	John Lohmeyer

To:		Membership of X3T10

From:		Bill Ham, SPI-2 Technical Editor

Larry Lamers, Vice-chair X3T10

John Lohmeyer, Chair X3T10

Subject:	Minutes of SPI-2 Working Group

August 13-14, 1996 -- Milpitas, CA

Agenda

1. Opening Remarks

2. Approval of Agenda

3. Attendance and Membership

4. LVD Topics

4.1 Hot Plugging [Ham]

4.2 Universal Backplane [Ham/Wallace]

4.3 Proposed corrections and clarifications regarding Release Glitches (96-226r0) [Lohmeyer]

4.4 SPI-2 Document Review (X3T10/1142D) [Ham]

5. High-Voltage Differential Fast-40 (96-190) [Gingerich]

6. Meeting Schedule

7. Adjournment

Results of Meeting

# Opening Remarks

John Lohmeyer, the X3T10 Chair, called the meeting to order at 9:00 a.m., Tuesday August 13, 1996.  He thanked Joe Molina of Technology Forums for hosting the meeting.

As is customary, the people attending introduced themselves and a copy of the attendance list was circulated.

# Approval of Agenda

The agenda was approved with the following additions:

# Attendance and Membership

Attendance at working group meetings does not count toward minimum attendance requirements for X3T10 membership.  Working group meetings are open to any person or organization directly and materially affected by X3T10’s scope of work.  The following people attended the meeting:

Name          S        Organization         Electronic Mail Address

---------------------- -- ------------------------- --------------------------

__ People Present

Status Key:  P    -  Principal

A,A# -  Alternate

O    -  Observer

L    -  Liaison

V    -  Visitor

# LVD Topics

## Hot Plugging [Ham]

Clarified the non-support for CASE 4.  Added notes regarding HVD/LVD SE mixing.

Need to expose the need for a variable impedance contact (a soft landing) to use for Case 4 hot plugging to connector folks.

Add Bill's picture of how it works.  One side of the contact is resistive not metallic.  Add a note to the email version to indicate that the illustration is only in the pdf file.

Pre-charge power

Pre-charge signals

Half-charge

## Universal Backplane [Ham/Wallace]

Little discussion, Dean Wallace was not present so his action item on a feasibility study was not available.

## Proposed corrections and clarifications regarding Release Glitches (96-226r0) [Lohmeyer]

for JBL to fill in.

## SPI-2 Document Review (X3T10/1142D) [Ham]

Bill stepped through the remaining part of the draft that was not covered at the previous meeting.  A number of corrections and adjustments were made.  The draft still needs a thorough editorial review.

# SPI-2 Single-ended Termination (X3T10/1142D) [Milligan]

Gene Milligan raised an issue (see 96-222) on how the single-ended termination driver current is specified.  Specifically the 48 ma max at 0.2 V d.c. seems to outlaw the existing SCSI-2 active termination.

Dean Wallace, Louis Grantham, and Paul Aloisi, are working on a proposal that would address the issue of keeping the terminator within the driver specifications.

# High-Voltage Differential Fast-40 (96-190) [Gingerich]

Kevin's stuff includes a redefinition of the timings to allow HVD.

Modify the DIFFSENS - Bill argued that burdening the LVD with the cost of meeting higher common mode test for the DIFFSENS signal is not justified.  He suggested reducing the voltage range of the test to -0.5 to 4.1 V d.c., the range for HVD is -4.5 to 12.0 V d.c.  This will result in two specifications in the working draft.

If the voltage on the input pins is not specified at 6 V then a differential device will blow the terminators.  A caution notes will be added to the draft: one on possible damage to SE/LVD devices when inserting an HVD device.  A driver circuit for HVD DIFFSENS will be added.

Delay skew and distortion skew gets replaced with a skew value for the setup and hold times figure.  The load imbalance may contribute up to 1.2 ns of skew.

# Meeting Schedule

The next meeting of SPI-2 Working Group is scheduled for September 9, 1996 in Natick, MA hosted by Digital Equipment.  Another meeting is scheduled for October 10, 1996 in St. Petersburg Beach, FL with X3T11 hosted by AMP.

# Adjournment

The meeting was adjourned at _:__ p.m. on Wednesday August 14, 1996.