X3T10/96-247 r0

Date:		November 15, 1996

To:		X3T10

From:		Larry Barnes, Symbios Logic

Subject:	Universal Backplanes

Commentary:

Further examination of the literature indicates that there are two basic types of coupled transmission line construction microstrip and stripline.

Microstrip is the construction with two parallel lines laid on the surface of a PCB with an AC ground plane.

Stripline is the construction with two parallel lines buried in a PCB between two AC ground planes.  Stripline has two configurations, edge-coupled, when the lines have the same Y coordinate and vary in placement in the X direction; broadside-coupled, when the lines have the same X coordinate and vary in placement in the Y direction.  Broadside-coupled lines have better coupling characteristics but are more difficult to bring to the surface without compromising electrical characteristics.  Edge-coupled lines are easier to bring to the surface without compromising electrical characteristics, but have a lower coupling coefficient.

The graphical data were from an article by Seymour Cohn, "Shielded Coupled-Strip Transmission Line", IRE Transactions on Microwave Theory and Techniques, October 1955.  This is the standard reference article for edge-coupled stripline.  He also presents a good description of the odd-even mode transmission characteristics.  Further information on coupled microstrip and broadside-coupled stripline can be found in the following articles respectively:  Garg, Ramesh and Bahl, "Characteristics of Coupled Microstrip", IEEE Transactions on Microwave Theory and Techniques", July 1977.    Bahl and Bhartia, "The Design of Broadside-Coupled Stripline Circuits", IEEE Transactions on Microwave Theory and Techniques", February 1981

The following data show that for coupled microstrip,  the tighter the coupling between the two lines the higher the even-mode differential impedance and single-ended impedance, for a give odd-mode differential impedance.  Remember that the differential impedance is 2 time the odd-mode impedance.

| Universal Backplane - Case 1 |  |  |  |
| --- | --- | --- | --- |
| Construction | Microstrip |  |  |
| Frequency | 240 MHz | Units | mil |
| Board Stackup |  |  |  |
| Layer # | Dielectric Constant | Dissipation Factor | Thickness |
| 1 | 1 | 0 | ì |
| 2 | 1 | 0 | 1.35 |
| 3 | 4.5 | 0 | 20.0 |
| Ground Plane | n/a | n/a | 1.35 |
| Conductors |  |  |  |
| Conductor # | Width | X Coordinate | Sigma |
| 1 | 24.0 | 0 | 5.800e+07 |
| 2 | 24.0 | 50.0 | 5.800e+07 |
| Ground Plane | n/a | n/a | 5.800e+07 |
| Coupled Characteristics of Modes |  |  |  |
| Mode | Zo | Effective er | Attenuation dB/mil |
| Odd | 54.59-j0.29 | 2.93 | 1.042e-05 |
| Even | 68.58-j0.26 | 3.47 | 8.296e-06 |
| Single Ended Characteristics of Conductors |  |  |  |
| Line # | Zo | Delay ns/mil | Attenuation dB/mil |
| 1 | 61.48-j0.27 | 1.525e-04 | 9.255e-06 |
| 2 | 61.48-j0.27 | 1.525e-04 | 9.255e-06 |

| Universal Backplane - Case 2 |  |  |  |
| --- | --- | --- | --- |
| Construction | Microstrip |  |  |
| Frequency | 240 MHz | Units | mil |
| Board Stackup |  |  |  |
| Layer # | Dielectric Constant | Dissipation Factor | Thickness |
| 1 | 1 | 0 | ì |
| 2 | 1 | 0 | 6.0 |
| 3 | 4.5 | 0 | 20.0 |
| Ground Plane | n/a | n/a | 1.20 |
| Conductors |  |  |  |
| Conductor # | Width | X Coordinate | Sigma |
| 1 | 15.0 | 0 | 5.800e+07 |
| 2 | 15.0 | 35.0 | 5.800e+07 |
| Ground Plane | n/a | n/a | 5.800e+07 |
| Coupled Characteristics of Modes |  |  |  |
| Mode | Zo | Effective er | Attenuation dB/mil |
| Odd | 57.88-j0.36 | 2.56 | 1.114e-05 |
| Even | 85.24-j0.40 | 3.24 | 9.405e-06 |
| Single Ended Characteristics of Conductors |  |  |  |
| Line # | Zo | Delay ns/mil | Attenuation dB/mil |
| 1 | 71.03-j0.38 | 1.467e-04 | 1.018e-05 |
| 2 | 71.03-j0.38 | 1.467e-04 | 1.018e-05 |

| Universal Backplane - Case 3 |  |  |  |
| --- | --- | --- | --- |
| Construction | Microstrip |  |  |
| Frequency | 240 MHz | Units | mil |
| Board Stackup |  |  |  |
| Layer # | Dielectric Constant | Dissipation Factor | Thickness |
| 1 | 1 | 0 | ì |
| 2 | 1 | 0 | 2.7 |
| 3 | 4.6 | 0 | 20.0 |
| Ground Plane | n/a | n/a | 1.2 |
| Conductors |  |  |  |
| Conductor # | Width | X Coordinate | Sigma |
| 1 | 12.0 | 0 | 5.800e+07 |
| 2 | 12.0 | 21.0 | 5.800e+07 |
| Ground Plane | n/a | n/a | 5.800e+07 |
| Coupled Characteristics of Modes |  |  |  |
| Mode | Zo | Effective er | Attenuation dB/mil |
| Odd | 54.85-j0.46 | 2.55 | 1.481e-05 |
| Even | 105.94-j0.51 | 3.24 | 9.676e-06 |
| Single Ended Characteristics of Conductors |  |  |  |
| Line # | Zo | Delay ns/mil | Attenuation dB/mil |
| 1 | 77.68-j0.46 | 1.517e-04 | 1.182e-05 |
| 2 | 77.68-j0.46 | 1.517e-04 | 1.182e-05 |