+-----------------------------------+

|  d  |  I  |  g  |  I  |  t  |  a  |  l  |

+-----------------------------------+

DATE:  Oct 10, 1996

FROM:  Bill Ham

Storage Bus Technical Office		`			LOC/MS:	SHR3-2/W8

334 South Street						DEPT:		STORAGE

Shrewsbury, MA 01545						EXT PH:	508 841-2629

DTN PH:	237-2629

EXT FAX:  	508 841-5266

DTN FAX:	237-5266

TO: X3T10

SUBJECT:  Case 4 hot plugging for SPI-2

The following wording is suggested for the Case 4 hot plugging section of SPI-2:

Case 4 addition and removal of devices shall follow SPI-1 case 4 with the exception that one or more of the following may be needed to ensure integrity in extreme conditions:

.Precharge the signal lines to 1.2V d.c. prior to contacting the active signal line

.use a staged resistance contact on the signal lines where the pin on the signal line on the device being inserted contacts a non-metallic resistance at the same potential as the active signal line prior to making the metal to metal contact with the signal pin

.use a filter on the data lines that prevents indicating a state change unless the detected state change lasts for at least 2 ns and a filter on the REQ and ACK lines that prevent responding to detected state changes for at least 5 ns after detecting a state changes (both assertion and negation)