Accredited Standards Committee*

X3, Information Technology

Doc. No.:	X3T10/96-xxx R0

Date:	October 15, 1996

Project:	1142-D

Ref. Doc.:

Reply to:	Mr. John Lohmeyer

Symbios Logic Inc.

4420 ArrowsWest Dr.

Colo Spgs, CO 80907

(719) 533-7560

To:		X3T10 Membership

From:		John Lohmeyer, X3T10 Chair

Larry Lamers, X3T10 Vice Chair

Subject:	SPI-2 Working Group -- October 10, 1996

St. Petersburg Beach, FL

A small SPI-2 Working Group meeting was held October 10, 1996 in St. Petersburg Beach, FL hosted by Charles Brill of AMP, Inc.  John Lohmeyer thanked Chuck for hosting the meeting and the exciting weather (TS Josephine) earlier in the week.

Attendance at working group meetings does not count toward minimum attendance requirements for X3T10 membership.  Working group meetings are open to any person or organization directly and materially affected by X3T10’s scope of work.  The following people attended the meeting:

Name          S        Organization         Electronic Mail Address

---------------------- -- ------------------------- -------------------------

Mr. Lawrence J. Lamers A  Adaptec, Inc.             ljlamers@aol.com

Mr. Michael Wingard    O  Amphenol Interconnect     mikwingard@aol.com

Mr. Louis Grantham     P  Dallas Semiconductor      grantham@dalsemi.com

Dr. William Ham        A# Digital Equipment Corp.   ham@subsys.enet.dec.com

Mr. John Lohmeyer      P  Symbios Logic Inc.        john.lohmeyer@symbios.com

Mr. Paul D. Aloisi     P  Unitrode Corporation      aloisi@unitrode.com

6 People Present

Status Key:  P    -  Principal

A,A# -  Alternate

O    -  Observer

L    -  Liaison

V    -  Visitor

The group focused on identifying the integration issues of merging the SPI, Fast-20, SIP, and SPI-2 Rev 11 documents into a unified SPI-2 document.  Larry Lamers prepared the following issue list to document the meeting results:

SPI-2 Unified Document (SPUD) Integration Issues

- Active negation - map of existance (see X3T10/95-295); map of intensity (use F20 envelope)

- SE termination, max/min current & sinking of current (see X3T10/96-222r1); suggest min current 20 milliamps at 0.2 v D.C.and a max of 25.4 milliamps at 0.2 v D.C.;  don't require driver to sink this to avoid re-qualify; add a min at 0.5 v D.C.; how to deal with non-linear terminators; do we need a duty-cycle spec? suggest 20 asserted signals for wide, 12 on narrow, 37 on 32-bit.  Add exception for a contained bus (e.g., laptop) of less than 0.3 meters. Add a 12.5 pf max capacitance for terminator.

- Leakage spec increased for LVD multi-mode drivers to 20 micro-amps - Resolve by specifing one for each driver type.

- Latching & counting - require that they be a monatomic action for hot plugging - possibly an annex or implementation note.

- Ground - power, logic & ground drivers (25-pin power & ground connected (SFF-8040)), signal ground and ground ground for tables defining SE signals.

- SE Receiver - hysterisis (.3), input levels (adopt F20), pin leakage (same), glitch filtering (enable of first detection of a transition for 'x' duration).  Need an algorythm to determine 'x'.

- SE Drivers - slew rate (use F20 specs);

- 16 or 32 devices? 32-bit data path? Is it a single segment draft?  Should expanders be included?  Suggest that SPI-2 is 16 loads per segment max; allow 32-bit data path and 32 SCSI IDs addressability when using VHDCI connectors with primary and secondary cables

- Case 4 hot plugging - requires SCA-2 connectors to allow precharge, a resistive contact may also work, needs research to determine max distrubance.

- Cable specifications - impedance (loaded & unloaded), skew, wire gauge, attenuation

- Add Q-cable pinouts

- Micro SCSI pinouts

The group discussed some possible solutions for several of the issues.  Since the group was quite small, the resolution of these issues was deferred to the Palm Springs meeting November 4th, where a larger group is expected.

Bill Ham prepared X3T10/96-252 on Case 4 hot plugging for SPI-2.  It outlines three possible methods of achieving data integrity with LVD case 4 hot plugging.  This document (96-252r0.doc) will be placed on the SCSI BBS and on the ftp site (ftp://ftp.symbios.com/pub/standards/io/x3t10/document.96/96-252r0.doc).

The next meetings of the SPI-2 Working Group are:  November 4, 1996 in Palm Springs, CA and December 5, 1996 in Bloomington, MN (with X3T11).