ACTIVE NEGATION MAP FOR

SPI-2 (SE, HVD and LVD)

(FUNCTION ON BUS: ACTIVE NEGATION REQUIREMENTS)

| SERVICE* | ACK | REQ | DATA | PARITY | BSY | SEL | RST | ATN | I/O, C/D, MSG |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SCAM | X | X | I or T:N | X | I or T:N | T:N | X | X | I or T:N |
| BUS FREE | X | X | X | X | X | X | X | X | X |
| ARBITRATION | X | X | ID:N | P:N | A:N | W:N | X | X | X |
| SELECTION | X | X | I:O | I:O | I&T:N | I:N | X | I:O | X |
| RESELECTION | X | X | T:O | T:O | I&T:N | T:N | X | I:O | T:O |
| COMMAND | I:O | T:O | I:O | I:O | T:N | X | X | I:O | T:O |
| DATA IN | I:R | T:R | T:R | T:R | T:N | X | X | I:O | T:O |
| DATA OUT | I:R | T:R | I:R | I:R | T:N | X | X | I:O | T:O |
| STATUS | I:O | T:O | T:O | T:O | T:N | X | X | I:O | T:O |
| MESSAGE IN | I:O | T:O | T:O | T:O | T:N | X | X | I:O | T:O |
| MESSAGE OUT | I:O | T:O | I:O | I:O | T:N | X | X | I:O | T:O |
| RESET | X | X | X | X | X | X | I or T:N | X | X |

ABBREVIATIONS:

FUNCTIONS:

I = INITIATOR

T = TARGET

X = NONE  (HIGH IMPEDANCE OUTPUT)

ID = SCSI ID

A = ALL

W = WINNER OF ABRITRATION

P = FUNCTION NOT REQUIRED

ASSERTION PERMITTED FROM ALL - SEE SECTION XXXXXX

ACTIVE NEGATION REQUIREMENTS:

N = ACTIVE NEGATION NOT ALLOWED (OR-TIED)

R = ACTIVE NEGATION REQUIRED

O = ACTIVE NEGATION OPTIONAL (NO HARM IF USED)

D = ACTIVE NEGATION RECOMMENDED (PERFORMANCE BENEFIT LIKELY)

* SERVICES ARE RELATED TO THE PHASES IN SCSI-2