10/16/96									ISO/IEC SC25/WG 4 LO3

G.E. Milligan

Seagate Technology Inc.

OKM 251

P.O. Box 12313

Tel: 405-324-3070

Fax: 405-324-3794

TO:	Larry Lamers		408-957-7193

John Lohmeyer		719-533-7036

Subject:	Copy of London 3 errata on SCSI-2

Errata on ISO/IEC 9316:1995

SCSI-2 JIS draft committee found the following technical errors:

1) Inch-mm conversion errors:

1 inch shall be 25.4 mm

| Fig. Item | ISO/IEC 9316:1995 |  | Correct | Note |
| --- | --- | --- | --- | --- |
|  | mm | in | mm |  |
| Fig 2 A8 | 0,60+- 0,03 | 0,024 +/- 0,001 | 0,61 +/- 0,03 | Both 50 and 68 pin connectors |
| Fig 2 A13 | 5,15 + 0,15 | 0,023 +/- 0,006 | 5,16 +/- 0,15 | Both 50 and 68 pin connectors |
| Fig 3 D9 | 6,09 | 0,240 | 0,610 |  |
| Fig 6 A10 | 0,60 +/- 0,03 | 0,024 +/- 0,001 | 0,61 +/- 0,03 | Both 50 and 68 pin connectors |

2) Command name error

A command name is not found in the Command list.

Subclause 15.1 is GET WINDOW PARAMETERS and should be GET WINDOW.

3) Description error

There is an inconsistent description in subclause 16.2.3 (Just under Table 303, page 343).

"A link bit of zero indicates a non-linked command; if the scan is satisfied, the command shall be terminated with an INTERMEDIATE-CONDITION MET status…

A link bit of one indicates that a command is linked to the MEDIUM SCAN command; if the scan is satisfied, INTERMEDIATE-CONDITION MET status is returned and next command is executed…"

It shall be as follows:

"A link bit of zero indicates a non-linked command; if the scan is satisfied, the command shall be terminated with an CONDITION MET status…

A link bit of one indicates that a command is linked to the MEDIUM SCAN command; if the scan is satisfied, INTERMEDIATE-CONDITION MET status is returned and next command is executed…"

Best regards,

Munemasa Mori

JTC1/SC25/WG4 and JTC1/SC18/WG9 of Japan