X3T10/97-127 r0

BSR X3.304-199x PR Comment #1

To: Deborah J. Donovan

From: Robert Reisch, Eastman Kodak

Subject: Kodak Public review comments for X3.304:199X, SCSI-3 Multimedia

Commands.

1.0  Page 114  Track Size for stamped media is not defined. On page 114

add the following:

For the note above Table 147 change from:

NOTE:  The Track Size number may not be exact for the tracks that do not

have a PMA entry.

To:

NOTE:  The Track Size number may not be exact for the tracks that do not

have a PMA entry.  The track size of stamped media is calculated as

follows:

TrackSize of track n = (start of track n+1) - (start of track n)

2.0  Page 37 the read CD command when EDC ECC is returned it is not

clear weather the data is ECC corrected. To clarify this make the

following changes:

Current specification reads:

The EDC and ECC bit, when set to one, indicates that the EDC and ECC

(L-EC) field shall be included in the data stream. For Mode 1 CD format,

this will include the 8 bytes of pad data.

Changed to:

The EDC and ECC bit, when set to one, indicates that the EDC and ECC

(L-EC) field shall be included in the data stream. The EDC and ECC data

returned shall be after L-EC has been applied. For Mode 1 CD format,

this will include the 8 bytes of pad data.

3.0  Table 37 Change the Sub Channel data selection value 011b from

reserved to Vendor Specific and optional. This will allow the

specification to meet unique customer needs for the format of

sub-channel data.

4.0A clarification of Table 111 Data Block Type Codes. Make the

following changes for the definition field:

From:

Raw data with P and Q sub-channel

2352 bytes of raw  data,

16 bytes buffer for Q sub-channel:

Bytes 0..9 are Q sub-channel data

Bytes 10..11 are Q sub-channel EDC

Bytes 12..14 are zero

Byte   15, most significant bit has state of P sub-channel bit

(not valid for write type = packet)

To:

Raw data with P and Q sub-channel

2352 bytes of raw  data,

16 bytes buffer for Q sub-channel:

Bytes 0..9 are Q sub-channel data

Bytes 10..11 are Q sub-channel EDC

Bytes 12..14 are zero

Byte   15, most significant bit has state of P sub-channel bit

(not valid for write type = packet)

(Q sub-channel data is in binary format see table 38)