To: 		T10 Technical Committee

- From:		Rob Elliott, HP (elliott@hp.com)

Date:		6 May 2003

Subject: 	T10/03-186r0 SAS-1.1 Transport layer retries

- Revision History

Revision 0 (6 May 2003) first revision

- Related Documents

sas-r03g – Serial Attached SCSI revision 3g

- Overview

In SAS-1, errors transmitting frames result in either the logical unit terminating the command with CHECK CONDITION status, or the application client aborting the command.

Possible responses to sending a frame:

- frame arrives; ACK arrives

- frame arrives with CRC error; NAK arrives

- frame arrives; ACK lost

- frame arrives; NAK lost

- frame lost

Possible errors:

Depending on the type of frame, different things happen in SAS-1.1:

|  | COMMAND or TASK (I to T) | XFER_RDY (T to I) | RESPONSE (T to I) | read DATA (T to I) | write DATA (I to T) |
| --- | --- | --- | --- | --- | --- |
| 1. Frame arrives OK; ACK arrives OK | Target runs the command after sending ACK. | Initiator replies with write DATA frame(s). | Both finish the command. Initiator can reuse tag after evidence of target progression. | Move on to more data or RESPONSE (or XFER_RDY for bidi commands) | Move on to more data, XFER_RDY, or RESPONSE (or read DATA for bidi commands) |
| 2. Frame arrives with CRC error; NAK arrives OK | Target idle after sending NAK. Initiator sees the NAK and can resend the command. | Target terminates command. | Target can resend with retransmit=1. | Target terminates command | Initiator aborts command |
| 3. Frame arrives OK; ACK lost | Target runs the command after sending ACK. I to T direction hung interlocked. Initiator detects ACK/NAK timeout in 1 ms. T to I direction could deliver DATA, XFER_RDY, or RESPONSE frames in that time. Initiator cannot tell between ACK lost, NAK lost, and frame lost w/o using QUERY TASK. | T to I hung interlocked. Target detects ACK/NAK timeout in 1 ms. I to T direction could deliver write DATA frames in that time. Target cannot tell between ACK lost, NAK lost, and frame lost. | T to I hung interlocked. Target detects ACK/NAK timeout in 1 ms. I to T direction not supposed to deliver new command with the same tag yet. Target resends with retransmit=1 in a new connection. | Subsequent ACKs/NAKs for data in flight are misassigned by the target. Target ACK/NAK timeout. Target terminates cmd. | Subsequent ACK/NAKs for data in flight are misassigned by the initiator. Initiator ACK/NAK timeout. Initiator aborts cmd. |
| 4. Frame arrives with CRC error; NAK lost (double error) | Target idle after sending NAK. I to T direction hung interlocked. Initiator detects ACK/NAK timeout in 1 ms. Initiator can resend cmd later. Initiator cannot tell between ACK lost, NAK lost, and frame lost w/o using QUERY TASK. | T to I hung interlocked. Target detects ACK/NAK timeout in 1 ms. Target cannot tell between ACK lost, NAK lost, and frame lost. | T to I hung interlocked. Target detects ACK/NAK timeout in 1 ms. Target resends with retransmit=1 in new connection. | Subsequent ACKs/NAKs for data in flight are misassigned by the target. Initiator sees data offset gap. Target ACK/NAK timeout. Target terminates command. Initiator aborts cmd because of gap (if subseq. DATA frames) | Subsequent ACK/NAKs for data in flight are misassigned by the initiator. Initiator ACK/NAK timeout. Initiator aborts cmd. Target terminates command because of gap (if subseq. DATA frames). Initiator Response Timeout may occur causing target to terminate cmd. |
| 5. Frame lost | Target idle. I to T direction hung interlocked. Initiator detects ACK/NAK timeout in 1 ms. Initiator can resend cmd later. Initiator cannot tell between ACK lost, NAK lost, and frame lost w/o using QUERY TASK. | T to I hung interlocked. Target detects ACK/NAK timeout in 1 ms. Target terminates cmd. Target cannot tell between ACK lost, NAK lost, and frame lost. | T to I hung interlocked. Target detects ACK/NAK timeout in 1 ms. Target resends with retransmit=1 in new connection. | Subsequent ACK/NAKs for data in flight are misassigned by the target. Initiator sees data offset gap. Target ACK/NAK timeout. Target terminates command. Initiator aborts cmd because of gap. | Subsequent ACK/NAKs for data in flight are misassigned by the initiator. Initiator ACK/NAK timeout. Initiator aborts cmd. Target terminates command because of gap (if subseq. DATA frames). Initiator Response Timeout may occur causing target to terminate cmd. |

Discussion in 5/6 SAS Protocol WG

Some backup applications will fail the whole backup if any of their commands are terminated with CHECK CONDITION.

Write commands – rewinding to the last XFER_RDY would be good

Read commands – rewinding to start of command may not be needed.  Restore software does better job of retrying.

Special recovery features not always wanted – per LUN controls needed

Mode page per LUN to enable/disable special recovery mode (not EMDP bit)

For write data problems

- Bit in XFER_RDY to tell the initiator that it should not get upset if its DATA frames encounter errors.

- Don’t want to depend on one XFER_RDY per write data frame

- Initiator sends a RECOVERY frame to target if it has trouble sending a DATA frame.  Target can then send a new XFER_RDY when it wants.

or probably simpler:

- Bit in XFER_RDY to tell the initiator to retry write DATA frames that encounter errors

- initiator resends DATA frames that are or may be bad (backing up to last XFER_RDY point if unsure about any frame in that data region)

- each frame has correct Data Offset and retransmit=1 (all resent frames have retransmit=1).

- If resent frames cross a RESPONSE frame, target discards subsequent frames (with a now-unknown tag)

- if resent frames cross an XFER_RDY, initiator has to accept it.  Target should use different Target Port Transfer Tag to help differentiate them.

- crossing only occurs if an ACK/NAK timeout occurs because an ACK was lost.

- receiving a RESPONSE frame or XFER_RDY does NOT serve as a link layer ACK for the outbound direction.  That frame must be honored, though.

For XFER_RDY problems

- if target sees write DATA frames without seeing an ACK for the XFER_RDY, target discards them

- target sends XFER_RDY with retransmit=1 if XFER_RDY has problems

- write DATA frames in response to an XFER_RDY with retransmit do not themselves have retransmit=1

- target shall change Target Port Transfer Tag between the XFER_RDYs

- initiator might stop transferring DATA for the first one, but doesn’t have to

For read data problems

- let target retry any read DATA frames with retransmit=1

- also add a “changing data pointer” bit sent when it knows it is rewinding

- target frames are monotonically increasing between those bits

- target must get ACK/NAK balance before going throwing away data that might have to be retransmitted

For command problems

- initiator could receive a RESPONSE frame before it gets an ACK for a command (if ACK is lost).  Command might be REWIND or ERASE.  Need to treat the command as successful somehow.

- Must accept RESPONSE at any time during this

- If ACK/NAK timeout occurs, send QUERY TASK (in new connection) to see if the command is there.

- If there, then assume it’s running fine.

- If not, then free to resend the command.

- If RESPONSE slipped in, don’t try to abort the command and don’t report it as aborted to the app client.

- if QUERY TASK is not supported, initiator will have to try ABORT TASK instead.  Tape drives should support QUERY TASK.

- Suggested Changes

TBD.